# TotalSheet Construction Standards & Specification Manual 
2026 Edition.docx

# Bibliography

- IS 456:2000 – Plain and Reinforced Concrete – Code of Practice, Bureau of Indian Standards (BIS), New Delhi.

- IS 1200 Series – Method of Measurement of Building and Civil Engineering Works (Parts 1–27: Earthwork, Concrete, Masonry, Finishes, etc.), BIS.

- IS 383:2016 – Specification for Coarse and Fine Aggregates from Natural Sources for Concrete, BIS.

- IS 10262:2019 – Concrete Mix Proportioning – Guidelines, Bureau of Indian Standards.

- NBC 2016 – National Building Code of India, Volumes 1 & 2, Bureau of Indian Standards.

- CPWD Delhi Schedule of Rates (DSR) 2023 – Central Public Works Department, Government of India.

- CPWD General Specifications 2022 – Volumes 1 & 2 – Standard specifications for civil works including concrete, masonry, finishing, and painting.

- Construction Cost Manual – Published by the Institute of Quantity Surveyors, India.

- NICMAR Handbook on Construction Practices – National Institute of Construction Management and Research (NICMAR), Pune.

- Manufacturers' Technical Data Sheets – Sika India, Roff, MYK Laticrete, Asian Paints, Berger, Ultratech, ACC Cement, JSW Cement, Ramco, etc.

- Total Sheet™ – Construction Estimation & Specification Platform. Proprietary material and labour consumption data sourced from live projects, estimation models, and specification codes integrated into the Total Sheet platform. Developed by Utna Systems Pvt Ltd, Mangalore. 🌐 https://totalsheet.app/

- Site Execution Feedback & Contractor Insights – Data verified through on-ground site reports, field engineers, and operational teams across diverse project typologies.

- CPWD Analysis of Rates (AOR) 2022 – Avaliable at https://cpwd.gov.in

- State PWD Schedule of Rates (SOR) – Latest (State-wise) Example (Karnataka): https://pwd.karnataka.gov.in

## Also we can subscribe to the BIM, Microsoft, for developer use to integrate AI into our website.

# Table of Contents

1	Construction Material & Workforce Consumption Standards	5

2	SPECIFICATION CREATION – FOUR STRUCTURED COST SECTIONS	6

3	MATERIAL USAGE RATIO	6

3.1	(Technical Material Requirement Section)	6

3.1.1	Detailed Description	6

3.1.2	Scope of Material Usage Ratio	7

3.1.3	Purpose in Rate Analysis	7

3.1.4	Strategic Importance	8

4	EXECUTION TEAM & INFRASTRUCTURE USE	8

4.1	(Direct Work Execution Section)	8

4.1.1	Detailed Description	8

4.1.2	Scope of Execution Team & Infrastructure	8

4.1.3	Purpose in Rate Analysis	9

4.1.4	Strategic Importance	9

5	OPERATIONAL EXPENSES	10

5.1	(Site Running Cost – Percentage-Based Allocation Method)	10

5.1.1	Detailed Description	10

5.1.2	Cost Base Application Principle	10

5.1.3	Scope of Operational Expenses	10

5.1.4	Strategic Importance	11

6	ADMINISTRATION COST	11

6.1	(Corporate Overhead – Percentage-Based Allocation Method)	11

6.1.1	Detailed Description	11

6.1.2	Cost Base Application Principle	12

6.1.3	Scope of Administration Cost	12

6.1.4	Strategic Importance	13

7	CONCLUSION	13

8	Construction Material & Workforce Consumption Standards	14

9	Earth Work	15

10	Earth Boring / Drilling for Piling Work	17

11	Pile Capping Work	21

12	Mould Column Casting (Flyover / Bridge / Megastructure)	23

13	Bridge Girder / Beam Casting – Using Ready Mix Concrete (RMC)	25

14	Earth Filling	26

15	RCC U-Type Drain Work	28

16	Road Concrete Work	32

17	PCC Work	34

18	Formwork / Shuttering	35

19	Steel Fabrication & Fixing	38

20	Fabrication Resource Consumption	40

21	Ready Mix Concreting Work	42

22	Concrete Work - Site Mix - Structure Element	44

23	Concrete Work - Site Mix -   Super Structure Element	47

24	Concrete Work – Site Mix -	49

25	Masonry Work	51

26	Cement Block Masonry	53

27	ACC Ash Block Masonry	55

28	Plastering Work – Internal	57

29	Plastering Work – External	59

30	Waterproofing Work	62

31	Floor Screeding Work	65

32	Floor Level Bed Work	67

33	Vacuum Dewatered Flooring- Site Mix	69

34	Vacuum Dewatered Flooring – Ready Mix	71

35	Cut Lintel Work	73

36	Tilling Work - Using Adhesive	75

37	Tiling Work- Using Cement Mortar	78

38	Granite Flooring Work	80

39	Sunken Filling	82

40	Painting Work	84

41	Anti-Termite Treatment	88

42	Granite Stone Masonry for Foundation	90

43	Site Clearance and Stripping of Top Soil	92

44	Excavation of Unsuitable Material	93

45	NTS Treatment on Exposed Surfaces	93

46	Backfilling with Laterite	95

47	NTS Treatment on Compacted Laterite	96

48	MIVAN (Aluminium) Shuttering	97

49	Internal Room Plumbing & Sanitary Module – Full Material & Labour Consumption	100

# Construction Material & Workforce Consumption Standards

A Comprehensive Reference for Execution Teams, Estimators & Project Planners

This document presents a standardised and data-driven compilation of construction material and manpower consumption rates across key building components. Designed to enhance project accuracy, cost estimation, procurement planning, and execution efficiency, this guide outlines consumption norms aligned with general construction practices under Indian standards.

Each table is structured per square meter or cubic meter and includes material quantities, labour deployment, equipment usage, and tooling guidance—calibrated for real-world site conditions and practical construction methods.

This consumption report is fully compatible with the Total Sheet application and can be directly utilised to create specification codes within the Total Sheet Specification Code Creator. All sections and dynamic fields are pre-structured, enabling you to use these codes seamlessly for BOQ generation, estimation, and running bill processing.

Total Sheet provides accurate reporting and real-time project consumption insights, allowing stakeholders to monitor, manage, and execute with precision.

Developed by UTNA SYSTEMS PVT LTD, this resource empowers contractors, engineers, architects, and developers with a native framework for delivering projects more efficiently.

# SPECIFICATION CREATION – FOUR STRUCTURED COST SECTIONS

Every work specification must be developed using a structured four-layer cost framework.
This methodology ensures financial clarity, technical accuracy, cost discipline, and sustainable profitability.

Each work item—whether RCC, masonry, plastering, finishing, plumbing, infrastructure, or specialized systems—must be analyzed through the following four sections before finalizing its rate.

# MATERIAL USAGE RATIO

## (Technical Material Requirement Section)

### Detailed Description

Material Usage Ratio defines the exact technical quantity of materials required to complete one measurable unit of work (such as 1 m³, 1 m², 1 running meter, or 1 metric ton).

This section is purely engineering-based and must be derived from:

- Approved drawings

- Mix design standards

- Technical specifications

- Execution methodology

- Site conditions

- Industry practice

It establishes the physical material requirement necessary to deliver the defined scope of work.

### Scope of Material Usage Ratio

This section includes:

- Cement quantity

- Sand quantity

- Coarse aggregate quantity

- Steel reinforcement quantity

- Bricks, blocks, or stone units

- Pipes, fittings, and fixtures

- Tiles, granite, and finishing materials

- Paints, primers, and coatings

- Waterproofing compounds and admixtures

- Adhesives and binding agents

- Grade variations (e.g., M20, M25, M30, etc.)

- Wastage allowance (typically 2%–10% depending on material type)

All quantities must be calculated per measurable unit and verified for practical site adjustments.

### Purpose in Rate Analysis

Material Usage Ratio enables calculation of the Material Cost per Unit by multiplying the required material quantities with their respective basic market rates.

Only basic cost values must be considered; tax components must be excluded at this stage.

### Strategic Importance

This section represents:

What physically goes into the work.

Incorrect material ratios directly result in cost loss and inaccurate rate analysis.
Material logic forms the technical foundation of the entire costing system.

# EXECUTION TEAM & INFRASTRUCTURE USE

### Detailed Description

This section defines the manpower and machinery resources required to convert materials into completed work.

Execution cost reflects the direct effort required for construction activity and must be based on realistic productivity standards and execution methodology.

It determines how materials are transformed into finished output.

### Scope of Execution Team & Infrastructure

This section may include:

- Skilled labour (mason, carpenter, bar bender, plumber, painter, fabricator, etc.)

- Unskilled labour or helpers

- Supervisory manpower directly allocated to the activity

- Machine hours (mixer, pump, excavator, crane, roller, etc.)

- Equipment usage duration

- Formwork or shuttering systems

- Staging and scaffolding

- Temporary supports and access systems

- Tool utilization

Execution cost must reflect actual productivity rates and realistic site performance.

### Purpose in Rate Analysis

Execution Cost per Unit is calculated by determining:

- Labour man-days × labour rate

- Machine hours × equipment rate

- Equipment ownership or depreciation allocation, where applicable

All calculations must be based on basic cost values excluding tax components.

### Strategic Importance

This section represents:

What it takes to physically execute the work.

Improper execution costing can lead to productivity mismatch, time overruns, labour shortages, and direct financial loss.

# OPERATIONAL EXPENSES

## (Site Running Cost – Percentage-Based Allocation Method)

### Detailed Description

Operational Expenses represent indirect but essential site-level costs required to sustain daily project operations.

These costs are not directly embedded within material or execution quantities but are necessary for enabling safe, continuous, and compliant construction activity.

Operational expenses are calculated on a percentage basis and must be applied to clearly defined cost bases.

Percentages must always be applied to basic cost values excluding tax components.

### Cost Base Application Principle

Operational expenses may be applied to:

- Material Cost (excluding tax)

- Execution Cost (excluding tax)

- Combined Material and Execution Cost (excluding tax)

Each operational component must be logically linked to the cost base it supports to avoid duplication or distortion.

### Scope of Operational Expenses

Operational expenses may include:

- Transportation and logistics

- Diesel and fuel consumption

- Electricity and water supply

- Site security

- Safety equipment and PPE

- Quality testing and inspections

- Waste disposal and housekeeping

- Temporary utilities and lighting

- Barricading and safety control

- Tool wear and minor equipment damage

- Site-level contingency allocation

These costs ensure site continuity and operational stability.

### Strategic Importance

Operational expenses represent:

The financial requirement to sustain and support the project site ecosystem.

A structured percentage allocation ensures transparent cost layering, controlled budgeting, and prevention of hidden site losses.

# ADMINISTRATION COST

## (Corporate Overhead – Percentage-Based Allocation Method)

### Detailed Description

Administration Cost represents corporate-level overhead required to sustain the organization beyond individual project activities.

These costs are not directly associated with a specific work item but must be proportionally recovered through structured rate analysis.

Administration cost is calculated as a percentage applied to selected cost layers, strictly on basic cost values excluding tax components.

### Cost Base Application Principle

Administration cost may be applied on:

- Material Cost only (excluding tax)

- Material and Execution Cost combined (excluding tax)

- Material, Execution, and Operational Cost combined (excluding tax)

The selected base must align with company financial policy and overhead recovery strategy.

The most comprehensive and stable approach is to apply administration cost on:

Material + Execution + Operational Cost
(excluding tax value)

### Scope of Administration Cost

Administration cost typically includes:

- Office rent and utilities

- Management and executive salaries

- Accounts and finance department expenses

- Legal and compliance fees

- Insurance policies

- IT infrastructure and systems

- Software and licensing costs

- Audit and taxation services

- Tender preparation expenses

- Corporate marketing expenses

- Travel and vehicle expenses

- General corporate overhead

These costs ensure business continuity and long-term sustainability.

### Strategic Importance

Administration cost represents:

The financial requirement to sustain the organization as a whole.

A structured administrative percentage ensures full overhead recovery, financial discipline, consistency in rate analysis, and protection of long-term organizational viability.

# CONCLUSION

The four-section structure—Material Usage Ratio, Execution Team & Infrastructure, Operational Expenses, and Administration Cost—creates a disciplined and transparent rate analysis framework.

This structured methodology ensures:

- Accurate estimation

- Logical cost allocation

- Prevention of hidden losses

- Transparent BOQ preparation

- Financial stability at both project and corporate levels

- Sustainable profit planning

This framework establishes a complete financial control system for construction specification development.

# Construction Material & Workforce Consumption Standards

A Comprehensive Reference for Execution Teams, Estimators & Project Planners

This document presents a standardised and data-driven compilation of construction material and manpower consumption rates across key building components. Designed to enhance project accuracy, cost estimation, procurement planning, and execution efficiency, this guide outlines consumption norms aligned with general construction practices under Indian standards.

Each table is structured per square meter or cubic meter and includes material quantities, labour deployment, equipment usage, and tooling guidance—calibrated for real-world site conditions and practical construction methods.

This consumption report is fully compatible with the Total Sheet application and can be directly utilised to create specification codes within the Total Sheet Specification Code Creator. All sections and dynamic fields are pre-structured, enabling you to use these codes seamlessly for BOQ generation, estimation, and running bill processing.

Total Sheet provides accurate reporting and real-time project consumption insights, allowing stakeholders to monitor, manage, and execute with precision.

Developed by UTNA SYSTEMS PVT LTD, this resource empowers contractors, engineers, architects, and developers with a native framework for delivering projects more efficiently.

# Earth Work

| Unit | As Applicable (m³ / m² / RM / MT depending on work type) |
| --- | --- |
| Description | Execution as per approved drawings and project specifications. |
| Material Requirements | As per approved mix design / material specifications. |
| Labour Requirement | Skilled and unskilled labour as per consumption norms. |
| Machinery / Equipment | Relevant construction equipment and tools. |
| Standard Codes | IS 456:2000, IS 1200, IS 10262, CPWD Specifications, Relevant IRC / IS Codes |
| Quality Requirements | Compliance with tolerances, cube tests, material testing. |
| Testing Requirements | Cube test, field density test, alignment checks as applicable. |
| Safety Requirements | PPE, safe access, shoring, equipment safety compliance. |
| Remarks | Subject to site conditions and engineer approval. |

Description:
Earthwork involves the excavation, loading, transportation, and disposal of soil and rock materials to achieve desired levels and contours in construction projects. It forms the foundation of site preparation for structures, roads, and utilities, impacting levels, drainage patterns, and the stability of subsequent works. The methodology, manpower, and machinery requirements vary with soil classification and project conditions, requiring effective planning and execution for productivity, cost efficiency, and quality compliance.

Consumption Chart per 1 m³

| Soil Type | Excavation Method | Labor (Man-days) | Excavator (Hours) | Truck (Tippers) (6–10 m³ capacity) | Method of Construction |
| --- | --- | --- | --- | --- | --- |
| Soft Soil | Manual + Machine | 0.30 – 0.40 | 0.08 – 0.10 | 0.10 – 0.12 | Excavation is performed using a mix of manual tools and mechanical excavators. Manual labor handles trimming and inaccessible areas, while excavators undertake bulk removal. Soil is transported using tippers. |
| Medium Soil | Mostly Machine | 0.20 – 0.30 | 0.10 – 0.12 | 0.11 – 0.13 | Medium-dense soils are removed primarily by hydraulic excavators. Manual input is minimal, limited to trench edges or confined spaces. Excavated soil is loaded and shifted via trucks. |
| Hard Soil | Machine + Breaker | 0.25 – 0.35 | 0.12 – 0.15 (Excavator)0.03 – 0.05 (Breaker) | 0.12 – 0.14 | Hard strata are loosened using rock breakers prior to excavation. Excavators lift and load loosened material. Adequate care is taken to protect adjoining structures during the operation. |
| Hard Murum | Machine + Labor | 0.35 – 0.40 | 0.13 – 0.16 | 0.12 – 0.14 | Excavation in hard murum, a compact decomposed rock, is undertaken by excavators with manual support for finishing and shaping. Removal is done via tipper trucks. |
| Soft Rock | Breaker + Excavator | 0.35 – 0.50 | 0.15 – 0.18 (Excavator)0.05 – 0.07 (Breaker) | 0.13 – 0.16 | Soft rock is initially fractured using a rock breaker. Excavators then remove debris. Proper sequencing ensures safety and operational efficiency. |
| Hard Rock (Blasting) | Breaker / Blasting | 0.40 – 0.60 | 0.18 – 0.22 (Combined) | 0.15 – 0.18 | Controlled blasting is employed to fragment hard rock. In sensitive zones, heavy rock breakers are used. Post-blasting material is cleared mechanically and transported with trucks. Pre-approvals and safety protocols are mandatory. |

# Earth Boring / Drilling for Piling Work

| Unit | As Applicable (m³ / m² / RM / MT depending on work type) |
| --- | --- |
| Description | Execution as per approved drawings and project specifications. |
| Material Requirements | As per approved mix design / material specifications. |
| Labour Requirement | Skilled and unskilled labour as per consumption norms. |
| Machinery / Equipment | Relevant construction equipment and tools. |
| Standard Codes | IS 456:2000, IS 1200, IS 10262, CPWD Specifications, Relevant IRC / IS Codes |
| Quality Requirements | Compliance with tolerances, cube tests, material testing. |
| Testing Requirements | Cube test, field density test, alignment checks as applicable. |
| Safety Requirements | PPE, safe access, shoring, equipment safety compliance. |
| Remarks | Subject to site conditions and engineer approval. |

Bored Cast-In-Situ Piling (300–900 mm Dia) Using Ready Mix Concrete with Pump – Standard Description

Description:
Bored cast-in-situ piles ranging from 300 mm to 900 mm diameter are utilized for foundations requiring high axial, lateral, and uplift capacity across residential towers, commercial blocks, hospitals, bridges, and industrial facilities. These piles are installed using rotary rigs with auger or bucket drilling in all soil strata, including soft soil, medium rock, and weathered rock conditions.

The piles are concreted using Ready Mix Concrete (RMC) delivered via pump and tremie pipe, ensuring continuous, uniform, non-segregated concrete placement, reducing wastage, maintaining high workability (slump 180–200 mm), and achieving required load-bearing capacity with consistent quality.

Steel reinforcement cages are fabricated and lowered into the bore to match structural design requirements, ensuring adequate cover, overlap, and straightness, critical for structural integrity and load transfer. Tremie concreting is adopted for piles deeper than 3 m below ground level to prevent segregation and maintain columnar integrity.

Temporary casing pipes are deployed in collapse-prone strata or water-logged conditions to stabilize the bore during drilling and concreting. Vibrators are applied at the collar level to remove air pockets and ensure the top concrete interface is level for pile caps.

Consumption Chart (Per Meter Depth Per Pile – 3000 mm Dia)

| Component | M25 Grade (RMC) | M30 Grade (RMC) | M35 Grade (RMC) | M40 Grade (RMC) | Remarks |
| --- | --- | --- | --- | --- | --- |
| RMC Volume | 0.07 m³/m | 0.07 m³/m | 0.07 m³/m | 0.07 m³/m | For 300 mm dia, theoretical volume = 0.071 m³/m |
| Reinforcement (Fe500) | 8 – 10 kg/m | 8 – 10 kg/m | 8 – 10 kg/m | 8 – 10 kg/m | Cage: 4 bars + links per design |
| Water (for pump cleaning) | 3 – 5 L/m | 3 – 5 L/m | 3 – 5 L/m | 3 – 5 L/m | For flushing lines post pour |
| Mason/Fitter | 0.12 MD/m | 0.12 MD/m | 0.12 MD/m | 0.12 MD/m | Cage alignment and pour supervision |
| Helper | 0.10 MD/m | 0.10 MD/m | 0.10 MD/m | 0.10 MD/m | Support for cage, pipeline handling |
| Drilling Rig/Winch | 0.22 Hr/m | 0.22 Hr/m | 0.22 Hr/m | 0.22 Hr/m | Soil type dependent |
| RMC Pump Machine | 0.12 Hr/m | 0.12 Hr/m | 0.12 Hr/m | 0.12 Hr/m | Boom/line pump usage |
| Tremie Pipe | Required | Required | Required | Required | 150–200 mm dia tremie pipe |
| Temporary Casing | As needed | As needed | As needed | As needed | To maintain bore stability |
| Vibrator (collar concreting only if required) | 0.05 Hr/m | 0.05 Hr/m | 0.05 Hr/m | 0.05 Hr/m | For top 500 mm only |

Consumption Chart (Per Meter Depth Per Pile – 450 mm Dia)

| Component | M25 Grade (RMC) | M30 Grade (RMC) | M35 Grade (RMC) | M40 Grade (RMC) | Remarks |
| --- | --- | --- | --- | --- | --- |
| RMC Volume | 0.16 m³/m | 0.16 m³/m | 0.16 m³/m | 0.16 m³/m | Theoretical volume = 0.159 m³/m |
| Reinforcement (Fe500) | 15 – 18 kg/m | 15 – 18 kg/m | 15 – 18 kg/m | 15 – 18 kg/m | Typically 6 bars (T16/T20) + links |
| Water (for pump cleaning) | 5 – 6 L/m | 5 – 6 L/m | 5 – 6 L/m | 5 – 6 L/m | For flushing pipelines post pour |
| Mason/Fitter | 0.15 MD/m | 0.15 MD/m | 0.15 MD/m | 0.15 MD/m | For cage alignment, pour support |
| Helper | 0.12 MD/m | 0.12 MD/m | 0.12 MD/m | 0.12 MD/m | For cage handling, pipeline arrangement |
| Drilling Rig/Winch | 0.30 Hr/m | 0.30 Hr/m | 0.30 Hr/m | 0.30 Hr/m | Strata dependent; rocky strata requires more time |
| RMC Pump Machine | 0.15 Hr/m | 0.15 Hr/m | 0.15 Hr/m | 0.15 Hr/m | Boom/line pump based on site constraints |
| Tremie Pipe | Required | Required | Required | Required | 200–250 mm dia tremie for uninterrupted flow |
| Temporary Casing | As needed | As needed | As needed | As needed | Essential in loose strata for bore stability |
| Vibrator (collar concreting only if needed) | 0.05 Hr/m | 0.05 Hr/m | 0.05 Hr/m | 0.05 Hr/m | For top 500 mm only |

Consumption Chart (Per Meter Depth Per Pile – 600 mm Dia)

| Component | M25 Grade (RMC) | M30 Grade (RMC) | M35 Grade (RMC) | M40 Grade (RMC) | Remarks |
| --- | --- | --- | --- | --- | --- |
| RMC Volume | 0.28 m³/m | 0.28 m³/m | 0.28 m³/m | 0.28 m³/m | Theoretical volume: 0.283 m³/m |
| Reinforcement (Fe500) | 24 – 28 kg/m | 24 – 28 kg/m | 24 – 28 kg/m | 24 – 28 kg/m | 8–10 T20/T25 + links per design |
| Water (pump line cleaning) | 8 – 10 L/m | 8 – 10 L/m | 8 – 10 L/m | 8 – 10 L/m | For post-pour cleaning |
| Mason/Fitter | 0.18 MD/m | 0.18 MD/m | 0.18 MD/m | 0.18 MD/m | Cage alignment, tremie handling |
| Helper | 0.14 MD/m | 0.14 MD/m | 0.14 MD/m | 0.14 MD/m | Reinforcement & tremie assistance |
| Drilling Rig/Winch | 0.35 Hr/m | 0.35 Hr/m | 0.35 Hr/m | 0.35 Hr/m | May vary based on strata (rock/soil) |
| RMC Pump Machine | 0.18 Hr/m | 0.18 Hr/m | 0.18 Hr/m | 0.18 Hr/m | Boom/line pump used for deep piles |
| Tremie Pipe | Required | Required | Required | Required | 250–300 mm dia tremie pipe |
| Temporary Casing | As needed | As needed | As needed | As needed | For water-bearing/loose strata stability |
| Vibrator (collar level) | 0.05 Hr/m | 0.05 Hr/m | 0.05 Hr/m | 0.05 Hr/m | Only for top 0.5 m if required |

Consumption Chart (Per Meter Depth Per Pile – 750 mm Dia)

| Component | M25 Grade (RMC) | M30 Grade (RMC) | M35 Grade (RMC) | M40 Grade (RMC) | Remarks |
| --- | --- | --- | --- | --- | --- |
| RMC Volume | 0.44 m³/m | 0.44 m³/m | 0.44 m³/m | 0.44 m³/m | Theoretical volume: 0.442 m³/m |
| Reinforcement (Fe500) | 28 – 34 kg/m | 28 – 34 kg/m | 28 – 34 kg/m | 28 – 34 kg/m | Typically 8–12 T25/T28 + links per design |
| Water (pump cleaning) | 10 – 12 L/m | 10 – 12 L/m | 10 – 12 L/m | 10 – 12 L/m | For post-pour cleaning |
| Mason/Fitter | 0.20 MD/m | 0.20 MD/m | 0.20 MD/m | 0.20 MD/m | Cage alignment, tremie handling |
| Helper | 0.15 MD/m | 0.15 MD/m | 0.15 MD/m | 0.15 MD/m | Reinforcement & tremie assistance |
| Drilling Rig/Winch | 0.40 Hr/m | 0.40 Hr/m | 0.40 Hr/m | 0.40 Hr/m | Varies based on strata (rock/soil) |
| RMC Pump Machine | 0.20 Hr/m | 0.20 Hr/m | 0.20 Hr/m | 0.20 Hr/m | Boom/line pump with tremie operation |
| Tremie Pipe | Required | Required | Required | Required | 300–350 mm dia tremie pipe |
| Temporary Casing | As needed | As needed | As needed | As needed | For loose strata/water table zones |
| Vibrator (collar level) | 0.05 Hr/m | 0.05 Hr/m | 0.05 Hr/m | 0.05 Hr/m | For top 0.5 m if required |

Consumption Chart (Per Meter Depth Per Pile – 900 mm Dia)

| Component | M25 Grade (RMC) | M30 Grade (RMC) | M35 Grade (RMC) | M40 Grade (RMC) | Remarks |
| --- | --- | --- | --- | --- | --- |
| RMC Volume | 0.64 m³/m | 0.64 m³/m | 0.64 m³/m | 0.64 m³/m | Theoretical volume: 0.636 m³/m |
| Reinforcement (Fe500) | 32 – 38 kg/m | 32 – 38 kg/m | 32 – 38 kg/m | 32 – 38 kg/m | Typically 10–14 T28/T32 with ties |
| Water (pump cleaning) | 12 – 15 L/m | 12 – 15 L/m | 12 – 15 L/m | 12 – 15 L/m | For pipe cleaning post-pour |
| Mason/Fitter | 0.22 MD/m | 0.22 MD/m | 0.22 MD/m | 0.22 MD/m | Cage placement, tremie handling |
| Helper | 0.16 MD/m | 0.16 MD/m | 0.16 MD/m | 0.16 MD/m | Supports rig and tremie activities |
| Drilling Rig/Winch | 0.45 Hr/m | 0.45 Hr/m | 0.45 Hr/m | 0.45 Hr/m | Varies by soil/rock strata |
| RMC Pump Machine | 0.22 Hr/m | 0.22 Hr/m | 0.22 Hr/m | 0.22 Hr/m | Boom/line pump operation |
| Tremie Pipe | Required | Required | Required | Required | 350–400 mm tremie suggested |
| Temporary Casing | As required | As required | As required | As required | For collapse-prone boreholes |
| Vibrator (collar level) | 0.06 Hr/m | 0.06 Hr/m | 0.06 Hr/m | 0.06 Hr/m | Collar consolidation only |

# Pile Capping Work

| Unit | As Applicable (m³ / m² / RM / MT depending on work type) |
| --- | --- |
| Description | Execution as per approved drawings and project specifications. |
| Material Requirements | As per approved mix design / material specifications. |
| Labour Requirement | Skilled and unskilled labour as per consumption norms. |
| Machinery / Equipment | Relevant construction equipment and tools. |
| Standard Codes | IS 456:2000, IS 1200, IS 10262, CPWD Specifications, Relevant IRC / IS Codes |
| Quality Requirements | Compliance with tolerances, cube tests, material testing. |
| Testing Requirements | Cube test, field density test, alignment checks as applicable. |
| Safety Requirements | PPE, safe access, shoring, equipment safety compliance. |
| Remarks | Subject to site conditions and engineer approval. |

Description:

Pile caps are reinforced concrete structural elements cast above pile groups to distribute superstructure loads uniformly while ensuring structural integration with columns or walls. Using Ready Mix Concrete (RMC) with pumping enhances quality, consistency, and speed of construction, reducing labour-intensive site mixing and enabling high-volume concreting with uniform slump control.

The operation involves:

- Placement of reinforcement cages as per design drawings.

- Shuttering using steel or ply systems for dimensional accuracy.

- Concrete pouring using transit mixers and concrete pump to reduce labor and cycle times.

- Vibrators for compaction to avoid honeycombing.

- Proper curing to achieve desired strength.

This workflow aligns with fast-tracked commercial, industrial, and infrastructure projects where large volumes and consistent quality are critical.

Consumption Chart per 1 m³

| Component | M30 Grade | M35 Grade | M45 Grade | Remarks |
| --- | --- | --- | --- | --- |
| Ready Mix Concrete (RMC) | 1.02 – 1.05 m³ | 1.02 – 1.05 m³ | 1.02 – 1.05 m³ | Includes pump wastage (2–5%) |
| Steel Reinforcement (TMT Fe500/Fe550) | 110 – 140 kg | 120 – 150 kg | 130 – 160 kg | As per design drawings |
| Concrete Pump Usage | 0.08 – 0.10 Hr | 0.08 – 0.10 Hr | 0.08 – 0.12 Hr | Transit mixer with boom pump |
| Vibrator Usage | 0.08 Hr | 0.08 Hr | 0.10 Hr | Needle/plate vibrators |
| Shuttering Carpenter | 0.15 – 0.18 MD | 0.15 – 0.18 MD | 0.18 – 0.20 MD | Formwork setup & stripping |
| Steel Fixer | 0.20 – 0.25 MD | 0.22 – 0.28 MD | 0.25 – 0.30 MD | Fabrication & tying |
| Mason (Pouring & Finishing) | 0.18 – 0.22 MD | 0.20 – 0.24 MD | 0.22 – 0.26 MD | Supervision & surface leveling |
| Helper | 0.15 – 0.18 MD | 0.16 – 0.20 MD | 0.18 – 0.22 MD | Concrete assistance |
| Water (for washing tools, curing) | 30 – 40 L | 30 – 40 L | 30 – 40 L | Minimal due to RMC usage |

# Mould Column Casting (Flyover / Bridge / Megastructure)

| Unit | As Applicable (m³ / m² / RM / MT depending on work type) |
| --- | --- |
| Description | Execution as per approved drawings and project specifications. |
| Material Requirements | As per approved mix design / material specifications. |
| Labour Requirement | Skilled and unskilled labour as per consumption norms. |
| Machinery / Equipment | Relevant construction equipment and tools. |
| Standard Codes | IS 456:2000, IS 1200, IS 10262, CPWD Specifications, Relevant IRC / IS Codes |
| Quality Requirements | Compliance with tolerances, cube tests, material testing. |
| Testing Requirements | Cube test, field density test, alignment checks as applicable. |
| Safety Requirements | PPE, safe access, shoring, equipment safety compliance. |
| Remarks | Subject to site conditions and engineer approval. |

Description:

Mould column casting in flyovers, bridges, and megastructures involves precision-controlled vertical concrete placement in heavy-duty formworks to achieve high strength, durability, and dimensional accuracy. These columns often support significant loads and require high-grade concrete (M35, M40, M45), high compaction, advanced shuttering systems, and reliable pumping methods.

The workflow involves:

- Heavy-duty steel/system formwork (H-frame, soldier systems, jump forms) for precision and reusability.

- High-grade concrete using Ready Mix Concrete (RMC) with boom pump for consistent quality.

- Reinforcement tying with congestion management to maintain clear cover.

- Controlled pour cycles to avoid cold joints and honeycombing.

- Use of internal vibrators for thorough compaction.

- Curing management, using water sprinkling or curing compounds.

This process is critical in infrastructure projects, metros, elevated corridors, and industrial structures where column quality directly affects the structural stability and service life of the project.

Consumption Chart per 1 m³

| Component | M35 Grade | M40 Grade | M45 Grade | Remarks |
| --- | --- | --- | --- | --- |
| Ready Mix Concrete (RMC) | 1.02 – 1.05 m³ | 1.02 – 1.05 m³ | 1.02 – 1.05 m³ | Includes pump wastage |
| Steel Reinforcement (TMT Fe500/550) | 130 – 170 kg | 140 – 180 kg | 150 – 190 kg | As per design, congested detailing |
| Concrete Pump Usage | 0.08 – 0.10 Hr | 0.10 – 0.12 Hr | 0.12 – 0.15 Hr | Boom pump with RMC |
| Vibrator Usage | 0.10 Hr | 0.12 Hr | 0.12 Hr | Needle vibrators (25–40 mm) |
| Shuttering Carpenter | 0.20 – 0.24 MD | 0.22 – 0.26 MD | 0.25 – 0.30 MD | Heavy-duty steel/form liner systems |
| Steel Fixer | 0.25 – 0.30 MD | 0.28 – 0.34 MD | 0.30 – 0.36 MD | Includes congestion handling |
| Mason (Pouring & Finishing) | 0.20 – 0.25 MD | 0.22 – 0.26 MD | 0.24 – 0.28 MD | Includes supervision during pouring |
| Helper | 0.18 – 0.22 MD | 0.20 – 0.24 MD | 0.22 – 0.25 MD | For pumping support |
| Water (for tools & curing) | 30 – 40 L | 30 – 40 L | 30 – 40 L | For minimal use, curing managed with RMC |

# Bridge Girder / Beam Casting – Using Ready Mix Concrete (RMC)

| Unit | As Applicable (m³ / m² / RM / MT depending on work type) |
| --- | --- |
| Description | Execution as per approved drawings and project specifications. |
| Material Requirements | As per approved mix design / material specifications. |
| Labour Requirement | Skilled and unskilled labour as per consumption norms. |
| Machinery / Equipment | Relevant construction equipment and tools. |
| Standard Codes | IS 456:2000, IS 1200, IS 10262, CPWD Specifications, Relevant IRC / IS Codes |
| Quality Requirements | Compliance with tolerances, cube tests, material testing. |
| Testing Requirements | Cube test, field density test, alignment checks as applicable. |
| Safety Requirements | PPE, safe access, shoring, equipment safety compliance. |
| Remarks | Subject to site conditions and engineer approval. |

Description:

Bridge girder and beam casting using Ready Mix Concrete (RMC) in flyovers, metro viaducts, and highway bridges involves high-precision, heavy-duty execution requiring high-grade concrete (M40, M45, M50) to ensure durability, high strength, and serviceability under dynamic loads.

Key operational phases:

- Shuttering (Formwork): Heavy-duty steel/truss/girder shuttering or modular system forms for large spans, camber accuracy, and smooth finish.

- Rebar Fabrication & Placement: High-congestion reinforcement including shear keys, stirrups, and post-tensioning ducts, requiring systematic fabrication and placement.

- Concreting: Using RMC via boom pump for high efficiency and quality, controlled pour rates to manage workability.

- Vibration: Needle or shutter vibrators for dense compaction.

- Post-Tensioning / Post-Strengthening: Cable or strand tensioning post curing, with grouting and anchoring for designed prestress levels.

- De-shuttering: After achieving required strength, using cranes and systematic removal procedures.

- Curing: Sprinkler system or curing compounds for uniform curing.

These activities directly affect timeline optimization, safety, and structural integrity in megastructure projects.

Consumption Chart per 1 m³

| Component | M40 Grade | M45 Grade | M50 Grade | Remarks |
| --- | --- | --- | --- | --- |
| Ready Mix Concrete (RMC) | 1.02 – 1.05 m³ | 1.02 – 1.05 m³ | 1.02 – 1.05 m³ | Including pump wastage |
| Steel Reinforcement | 150 – 180 kg | 160 – 190 kg | 170 – 200 kg | As per design, includes PT ducts |
| Concrete Pump Usage | 0.08 – 0.12 Hr | 0.10 – 0.14 Hr | 0.12 – 0.16 Hr | Boom pump with flexible pipe |
| Vibrator Usage | 0.12 Hr | 0.14 Hr | 0.15 Hr | Needle & shutter vibrators |
| Shuttering Carpenter | 0.22 – 0.26 MD | 0.25 – 0.30 MD | 0.28 – 0.32 MD | Heavy-duty steel shuttering |
| Steel Fixer | 0.28 – 0.32 MD | 0.30 – 0.36 MD | 0.34 – 0.40 MD | Includes rebar & PT cable fixing |
| Mason (Pouring & Finishing) | 0.22 – 0.26 MD | 0.24 – 0.28 MD | 0.26 – 0.30 MD | Manages pour alignment & finish |
| Helper | 0.20 – 0.24 MD | 0.22 – 0.26 MD | 0.24 – 0.28 MD | Assists in all phases |
| Water (Tools & Curing) | 30 – 40 L | 30 – 40 L | 30 – 40 L | For tools and initial curing |
| Post-Tensioning (Cable & Anchorage) | 2 – 3 kg/m | 2 – 3 kg/m | 2.5 – 3.5 kg/m | As per span & design |
| De-shuttering Time | 7 – 10 days | 7 – 12 days | 10 – 14 days | Based on strength gain |
| Post-Tensioning Grout | 0.5 – 0.8 kg/m | 0.5 – 0.8 kg/m | 0.6 – 1.0 kg/m | For duct grouting |

# Earth Filling

| Unit | As Applicable (m³ / m² / RM / MT depending on work type) |
| --- | --- |
| Description | Execution as per approved drawings and project specifications. |
| Material Requirements | As per approved mix design / material specifications. |
| Labour Requirement | Skilled and unskilled labour as per consumption norms. |
| Machinery / Equipment | Relevant construction equipment and tools. |
| Standard Codes | IS 456:2000, IS 1200, IS 10262, CPWD Specifications, Relevant IRC / IS Codes |
| Quality Requirements | Compliance with tolerances, cube tests, material testing. |
| Testing Requirements | Cube test, field density test, alignment checks as applicable. |
| Safety Requirements | PPE, safe access, shoring, equipment safety compliance. |
| Remarks | Subject to site conditions and engineer approval. |

Description:
Earth filling is a critical component in foundation and structural preparation, involving the systematic placement, moisture conditioning, and compaction of soil to achieve desired levels, stability, and load-bearing capacity. The methodology varies depending on the type of fill required—foundation, plinth, or mass—and includes specific machinery, labor effort, water requirement, and compaction techniques.

Consumption Chart per 1 m³

| Filling Type | Labor(Man-days/m³) | Excavator/Loader(Hours/m³) | Tippers (6–10 m³)(Trips/m³) | Compaction Equipment | Water(Liters/m³) |
| --- | --- | --- | --- | --- | --- |
| Foundation Filling | 0.25 – 0.30 | 0.10 – 0.12 | 0.12 – 0.15 | Plate Compactor (0.12 Hr/m³) | 25 – 30 L |
| Plinth Filling | 0.30 – 0.35 | 0.12 – 0.15 | 0.15 – 0.18 | Vibratory Roller / Plate (0.15 Hr/m³) | 30 – 35 L |
| Mass Filling | 0.20 – 0.25 | 0.08 – 0.10 | 0.10 – 0.12 | Static / Vibro Roller (0.10 Hr/m³) | 20 – 25 L |

# RCC U-Type Drain Work

| Unit | As Applicable (m³ / m² / RM / MT depending on work type) |
| --- | --- |
| Description | Execution as per approved drawings and project specifications. |
| Material Requirements | As per approved mix design / material specifications. |
| Labour Requirement | Skilled and unskilled labour as per consumption norms. |
| Machinery / Equipment | Relevant construction equipment and tools. |
| Standard Codes | IS 456:2000, IS 1200, IS 10262, CPWD Specifications, Relevant IRC / IS Codes |
| Quality Requirements | Compliance with tolerances, cube tests, material testing. |
| Testing Requirements | Cube test, field density test, alignment checks as applicable. |
| Safety Requirements | PPE, safe access, shoring, equipment safety compliance. |
| Remarks | Subject to site conditions and engineer approval. |

Description:

RCC U-Type drains are critical for stormwater and wastewater management in highways, industrial parks, smart cities, and megastructures. They provide:
✅ High discharge capacity
✅ Structural strength under heavy loads
✅ Long service life with minimal maintenance.

Execution involves excavation, levelling, shuttering, steel fixing, RMC/Site Manual Mix concreting with vibration, curing, and de-shuttering, ensuring speed, quality, and cost efficiency under lean construction workflows.

Consumption Chart (Per Running Meter) - Ready Mix Concrete

| Component | 600x600 | 600x900 | 900x900 | 1000x1000 | 1000x1200 |
| --- | --- | --- | --- | --- | --- |
| Concrete (RMC M25) | 0.14 – 0.16 m³ | 0.20 – 0.22 m³ | 0.28 – 0.30 m³ | 0.36 – 0.40 m³ | 0.42 – 0.46 m³ |
| Steel Reinforcement | 12 – 15 kg | 18 – 22 kg | 25 – 30 kg | 32 – 38 kg | 38 – 44 kg |
| Formwork (Shuttering) | 1.4 – 1.5 m² | 1.8 – 2.0 m² | 2.4 – 2.6 m² | 2.8 – 3.0 m² | 3.2 – 3.5 m² |
| Concrete Pump | 0.02 – 0.03 Hr | 0.03 – 0.04 Hr | 0.04 – 0.05 Hr | 0.05 – 0.06 Hr | 0.06 – 0.07 Hr |
| Vibrator Usage | 0.02 Hr | 0.03 Hr | 0.03 Hr | 0.04 Hr | 0.04 Hr |
| Carpenter (Shuttering) | 0.08 – 0.10 MD | 0.10 – 0.12 MD | 0.12 – 0.14 MD | 0.14 – 0.16 MD | 0.16 – 0.18 MD |
| Steel Fixer | 0.06 – 0.08 MD | 0.08 – 0.10 MD | 0.10 – 0.12 MD | 0.12 – 0.14 MD | 0.14 – 0.16 MD |
| Mason (Pouring & Finish) | 0.05 – 0.06 MD | 0.06 – 0.08 MD | 0.08 – 0.10 MD | 0.10 – 0.12 MD | 0.12 – 0.14 MD |
| Helper | 0.06 – 0.08 MD | 0.08 – 0.10 MD | 0.10 – 0.12 MD | 0.12 – 0.14 MD | 0.14 – 0.16 MD |
| Water (Curing & Wash) | 6 – 8 L | 8 – 10 L | 10 – 12 L | 12 – 15 L | 14 – 18 L |
| DE shuttering Time | 3 – 5 days | 4 – 6 days | 5 – 7 days | 5 – 7 days | 5 – 8 days |

Consumption Chart (Per Running Meter) – On Site Manual Mix Concrete

| Component | 600x600 | 600x900 | 900x900 | 1000x1000 | 1000x1200 |
| --- | --- | --- | --- | --- | --- |
| Concrete (M25 site mix) | 0.14 – 0.16 m³ | 0.20 – 0.22 m³ | 0.28 – 0.30 m³ | 0.36 – 0.40 m³ | 0.42 – 0.46 m³ |
| Cement (OPC/PPC) | 1.8 – 2.0 bags | 2.5 – 2.8 bags | 3.5 – 3.8 bags | 4.5 – 5.0 bags | 5.2 – 5.8 bags |
| Sand (Zone II) | 0.07 – 0.08 m³ | 0.10 – 0.11 m³ | 0.14 – 0.15 m³ | 0.18 – 0.20 m³ | 0.21 – 0.23 m³ |
| Jelly (20 mm) | 0.10 – 0.12 m³ | 0.14 – 0.15 m³ | 0.20 – 0.22 m³ | 0.26 – 0.28 m³ | 0.30 – 0.33 m³ |
| Water | 18 – 20 L | 24 – 28 L | 34 – 38 L | 42 – 48 L | 50 – 55 L |
| Steel Reinforcement | 12 – 15 kg | 18 – 22 kg | 25 – 30 kg | 32 – 38 kg | 38 – 44 kg |
| Formwork (Shuttering) | 1.4 – 1.5 m² | 1.8 – 2.0 m² | 2.4 – 2.6 m² | 2.8 – 3.0 m² | 3.2 – 3.5 m² |
| Mixer Machine (5–7 CFT) | 0.08 – 0.10 Hr | 0.10 – 0.12 Hr | 0.14 – 0.16 Hr | 0.18 – 0.20 Hr | 0.20 – 0.22 Hr |
| Vibrator Usage | 0.02 Hr | 0.03 Hr | 0.03 Hr | 0.04 Hr | 0.04 Hr |
| Carpenter (Shuttering) | 0.08 – 0.10 MD | 0.10 – 0.12 MD | 0.12 – 0.14 MD | 0.14 – 0.16 MD | 0.16 – 0.18 MD |
| Steel Fixer | 0.06 – 0.08 MD | 0.08 – 0.10 MD | 0.10 – 0.12 MD | 0.12 – 0.14 MD | 0.14 – 0.16 MD |
| Mason (Pouring & Finish) | 0.05 – 0.06 MD | 0.06 – 0.08 MD | 0.08 – 0.10 MD | 0.10 – 0.12 MD | 0.12 – 0.14 MD |
| Helper | 0.06 – 0.08 MD | 0.08 – 0.10 MD | 0.10 – 0.12 MD | 0.12 – 0.14 MD | 0.14 – 0.16 MD |
| DE shuttering Time | 3 – 5 days | 4 – 6 days | 5 – 7 days | 5 – 7 days | 5 – 8 days |

Applicable for Mass Road & Infrastructure Projects

Consumption Chart (Per Running Meter) - Ready Mix Concrete

| Component | 600x600 | 600x900 | 900x900 | 1000x1000 | 1000x1200 |
| --- | --- | --- | --- | --- | --- |
| Concrete (M25/30 RMC Pumped) | 0.14 – 0.16 m³ | 0.20 – 0.22 m³ | 0.28 – 0.30 m³ | 0.36 – 0.40 m³ | 0.42 – 0.46 m³ |
| Pumping Machine (Boom/Pipeline) | 0.02 Hr | 0.03 Hr | 0.04 Hr | 0.05 Hr | 0.06 Hr |
| Steel Reinforcement | 12 – 15 kg | 18 – 22 kg | 25 – 30 kg | 32 – 38 kg | 38 – 44 kg |
| Formwork (Shuttering) | 1.4 – 1.5 m² | 1.8 – 2.0 m² | 2.4 – 2.6 m² | 2.8 – 3.0 m² | 3.2 – 3.5 m² |
| Vibrator Usage | 0.02 Hr | 0.03 Hr | 0.03 Hr | 0.04 Hr | 0.04 Hr |
| Carpenter (Shuttering) | 0.08 – 0.10 MD | 0.10 – 0.12 MD | 0.12 – 0.14 MD | 0.14 – 0.16 MD | 0.16 – 0.18 MD |
| Steel Fixer | 0.06 – 0.08 MD | 0.08 – 0.10 MD | 0.10 – 0.12 MD | 0.12 – 0.14 MD | 0.14 – 0.16 MD |
| Mason (Finishing/Alignment) | 0.04 – 0.06 MD | 0.06 – 0.08 MD | 0.08 – 0.10 MD | 0.10 – 0.12 MD | 0.12 – 0.14 MD |
| Helper | 0.06 – 0.08 MD | 0.08 – 0.10 MD | 0.10 – 0.12 MD | 0.12 – 0.14 MD | 0.14 – 0.16 MD |
| Deshuttering Time | 2 – 3 days | 3 – 4 days | 4 – 5 days | 5 – 6 days | 6 – 7 days |
| Water (Curing) | 12 – 15 L | 18 – 22 L | 25 – 30 L | 32 – 38 L | 40 – 45 L |

# Road Concrete Work

PCC Base & Main Concrete Layers

| Unit | As Applicable (m³ / m² / RM / MT depending on work type) |
| --- | --- |
| Description | Execution as per approved drawings and project specifications. |
| Material Requirements | As per approved mix design / material specifications. |
| Labour Requirement | Skilled and unskilled labour as per consumption norms. |
| Machinery / Equipment | Relevant construction equipment and tools. |
| Standard Codes | IS 456:2000, IS 1200, IS 10262, CPWD Specifications, Relevant IRC / IS Codes |
| Quality Requirements | Compliance with tolerances, cube tests, material testing. |
| Testing Requirements | Cube test, field density test, alignment checks as applicable. |
| Safety Requirements | PPE, safe access, shoring, equipment safety compliance. |
| Remarks | Subject to site conditions and engineer approval. |

Description:
Road concrete work using Ready-Mix Concrete (RMC) ensures high quality, consistency, and speed of execution for mass road and infrastructure projects. A PCC (Plain Cement Concrete) base layer is applied first to stabilize the subgrade and control moisture, providing a firm, even surface for the subsequent main concrete layer. The main concrete layer (M25, M30, M35 grade) forms the durable wearing surface, capable of handling heavy traffic loads and environmental exposure.

For mass projects, RMC deployment reduces wastage, improves quality control, and speeds up timelines. Mechanized processes using concrete pumps, vibrators, laser levelers, paver screeds, and curing systems enhance productivity while ensuring finish quality and compaction.
Joint cutting within 8–12 hours post-placement is essential for crack control, and curing (water or compound) must continue for 7–14 days for strength development.

This methodology ensures long-lasting, low-maintenance roads, aligning with infrastructure scalability goals for smart cities, industrial corridors, and expressway networks.

Base Layer (PCC) for Road Concrete Work - Using M10 / M15 Grade, Ready-Mix Concrete

Consumption Chart per 1 m³

| Component | 100 mm | 150 mm | 200 mm | 250 mm | 300 mm |
| --- | --- | --- | --- | --- | --- |
| PCC Volume | 0.010 m³ | 0.015 m³ | 0.020 m³ | 0.025 m³ | 0.030 m³ |
| Cement Content (in RMC) | 3.5 – 4.0 kg | 5.0 – 6.0 kg | 7.0 – 8.0 kg | 8.5 – 10.0 kg | 10.5 – 12.0 kg |
| Pumping Machine (if used) | 0.002 Hr | 0.003 Hr | 0.004 Hr | 0.005 Hr | 0.006 Hr |
| Vibrator | 0.002 Hr | 0.003 Hr | 0.004 Hr | 0.005 Hr | 0.006 Hr |
| Mason (Leveling & Finishing) | 0.02 – 0.03 MD | 0.03 – 0.04 MD | 0.04 – 0.05 MD | 0.05 – 0.06 MD | 0.06 – 0.07 MD |
| Helper | 0.015 – 0.02 MD | 0.02 – 0.025 MD | 0.025 – 0.03 MD | 0.03 – 0.035 MD | 0.035 – 0.04 MD |
| Water (Curing) | 2 – 3 L | 3 – 4 L | 4 – 5 L | 5 – 6 L | 6 – 7 L |

Main Concrete Layer for Road Work - Using M25 / M30 / M35 Grade, Ready-Mix Concrete

Consumption Chart per 1 m³

| Component | 100 mm | 150 mm | 200 mm | 250 mm | 300 mm |
| --- | --- | --- | --- | --- | --- |
| Concrete Volume | 0.090 m³ | 0.135 m³ | 0.180 m³ | 0.225 m³ | 0.270 m³ |
| Cement Content (in RMC) | 38 – 44 kg | 58 – 66 kg | 76 – 88 kg | 96 – 110 kg | 114 – 132 kg |
| Steel (Tie Bars/Dowels, if applicable) | 0.5 – 1.0 kg | 0.8 – 1.5 kg | 1.0 – 2.0 kg | 1.5 – 2.5 kg | 2.0 – 3.0 kg |
| Pumping Machine | 0.015 Hr | 0.020 Hr | 0.025 Hr | 0.030 Hr | 0.035 Hr |
| Concrete Vibrator | 0.015 Hr | 0.020 Hr | 0.025 Hr | 0.030 Hr | 0.035 Hr |
| Road Paver/Screed Leveler | 0.020 Hr | 0.025 Hr | 0.030 Hr | 0.035 Hr | 0.040 Hr |
| Laser Leveler (if used) | 0.010 Hr | 0.015 Hr | 0.020 Hr | 0.025 Hr | 0.030 Hr |
| Mason (Spreading & Finishing) | 0.08 – 0.10 MD | 0.10 – 0.12 MD | 0.12 – 0.14 MD | 0.14 – 0.16 MD | 0.16 – 0.18 MD |
| Helper | 0.06 – 0.08 MD | 0.08 – 0.10 MD | 0.10 – 0.12 MD | 0.12 – 0.14 MD | 0.14 – 0.16 MD |
| Water (Curing) | 8 – 10 L | 12 – 14 L | 16 – 18 L | 20 – 24 L | 24 – 28 L |

# PCC Work

| Unit | As Applicable (m³ / m² / RM / MT depending on work type) |
| --- | --- |
| Description | Execution as per approved drawings and project specifications. |
| Material Requirements | As per approved mix design / material specifications. |
| Labour Requirement | Skilled and unskilled labour as per consumption norms. |
| Machinery / Equipment | Relevant construction equipment and tools. |
| Standard Codes | IS 456:2000, IS 1200, IS 10262, CPWD Specifications, Relevant IRC / IS Codes |
| Quality Requirements | Compliance with tolerances, cube tests, material testing. |
| Testing Requirements | Cube test, field density test, alignment checks as applicable. |
| Safety Requirements | PPE, safe access, shoring, equipment safety compliance. |
| Remarks | Subject to site conditions and engineer approval. |

Description:

Plain Cement Concrete (PCC) and Reinforced Cement Concrete (RCC) are essential structural and non-structural components in construction. Various concrete grades serve different purposes such as bedding, foundations, and structural works. Each mix is designed with specific proportions of cement, sand, and coarse aggregates, suitable for its designated strength and durability.

Consumption Chart per 1 m³

| Component | M5 (1:5:10) | M7.5 (1:4:8) | M10 (1:3:6) | M15 (1:2:4) | M20 (1:1.5:3) | Method of Construction |
| --- | --- | --- | --- | --- | --- | --- |
| Cement (Type) | PPC (Fly ash) | PPC (Fly ash) | PPC / OPC 33 | OPC 43 / PPC | OPC 53 (RCC Grade) | Use fresh cement, stored in dry conditions; batching by weight for accuracy. |
| Cement (Bags) | 2.6 Bags | 4.4 Bags | 6.0 Bags | 8.5 Bags | 10.5 Bags | Ensure correct batching using pre-calibrated boxes or weigh-batcher. |
| Cement (kg) | 130 kg | 220 kg | 300 kg | 425 kg | 525 kg | Follow IS code-based mix design as per structural requirement. |
| Sand (m³) | 0.50 m³ | 0.45 m³ | 0.43 m³ | 0.40 m³ | 0.42 m³ | Use clean, well-graded river/crushed sand. |
| Jelly / Coarse Agg. | 0.95 m³ (20 mm) | 0.90 m³ (20 mm) | 0.85 m³ (20 mm) | 0.80 m³ (20 mm) | 0.78 m³ (20 mm + 10 mm) | Use 10 mm + 20 mm graded crushed stone jelly. |
| Water (litres) | 160 – 180 L | 180 – 200 L | 190 – 210 L | 200 – 220 L | 210 – 230 L | Maintain proper water-cement ratio (w/c) to achieve workability and strength. |
| Mason (Man-days) | 0.25 | 0.25 – 0.30 | 0.30 | 0.35 | 0.40 | Skilled masons to supervise placing, compaction & finishing. |
| Helper (Man-days) | 0.40 | 0.50 | 0.55 | 0.60 | 0.65 | Helpers assist in loading, pouring, and leveling. |
| Mixing Machine (Hr) | 0.08 | 0.10 | 0.12 | 0.14 | 0.15 | Concrete to be mixed in mechanical mixers for uniformity. |
| Vibrator (Hr) | Not Required | Optional (0.05) | Optional (0.05) | 0.05 – 0.07 | 0.07 – 0.10 | Use needle vibrators to remove air voids in M15/M20. |

# Formwork / Shuttering

Formwork (Shuttering) – Material & Manpower Consumption per 1 m²

| Unit | As Applicable (m³ / m² / RM / MT depending on work type) |
| --- | --- |
| Description | Execution as per approved drawings and project specifications. |
| Material Requirements | As per approved mix design / material specifications. |
| Labour Requirement | Skilled and unskilled labour as per consumption norms. |
| Machinery / Equipment | Relevant construction equipment and tools. |
| Standard Codes | IS 456:2000, IS 1200, IS 10262, CPWD Specifications, Relevant IRC / IS Codes |
| Quality Requirements | Compliance with tolerances, cube tests, material testing. |
| Testing Requirements | Cube test, field density test, alignment checks as applicable. |
| Safety Requirements | PPE, safe access, shoring, equipment safety compliance. |
| Remarks | Subject to site conditions and engineer approval. |

Description:

Formwork (shuttering) is a temporary or permanent mould system used to support and shape concrete until it gains sufficient strength to support its weight and applied loads. The selection and deployment of formwork directly impact project timelines, quality of finishes, labor productivity, and cost efficiency across all construction verticals.

Formwork operations require precise planning of manpower, equipment, and shuttering material tailored to each structural element, ensuring alignment with concrete pour cycles and effective material turnaround for high-volume or high-rise projects. Selection between timber, plywood, steel, and system formworks depends on project scale, reusability targets, finish requirements, and site logistics strategy.

The following table outlines manpower and equipment requirements per 1 m² of shuttering across key structural elements for practical estimation and resource planning:

Consumption Chart per 1 m²

| Structural Element | Carpenter(Man-days/m²) | Helper(Man-days/m²) | Shuttering Material |
| --- | --- | --- | --- |
| Footing | 0.18 – 0.22 | 0.12 – 0.15 | Plywood + Planks (Timber/Steel) |
| Raft Slab | 0.20 – 0.25 | 0.15 – 0.18 | Waterproof Plywood + Props |
| Column | 0.25 – 0.30 | 0.18 – 0.22 | Steel/Plywood Box or System Forms |
| Beam | 0.30 – 0.35 | 0.20 – 0.25 | Plywood + Steel Staging |
| Slab | 0.28 – 0.32 | 0.18 – 0.22 | Plywood Sheets + H-Frame / Props |
| Plinth Beam | 0.22 – 0.26 | 0.15 – 0.18 | Plank + Plywood + Supports |
| Mass Shuttering Yard Work | 0.15 – 0.20 | 0.10 – 0.12 | Form Panels (Steel/Ply) |

Formwork (Shuttering) – Material & Manpower Consumption per 1 m³

Description:
Formwork operations for Plinth Beams, Columns, and Beams require calibrated material and manpower estimation aligned with pour cycles and structural volume. Providing this data per m³ of concrete work ensures precise labor budgeting, effective resource mobilization, and optimized productivity across high-volume pours, contributing directly to site efficiency and quality assurance.

Use these parameters for rate analysis, micro-scheduling, and BOQ integration within your site execution systems:

Consumption Chart per 1 m³

| Structural Element | Carpenter (Man-days/m³) | Helper (Man-days/m³) | Shuttering Material |
| --- | --- | --- | --- |
| Plinth Beam | 1.00 – 1.20 MD | 0.70 – 0.80 MD | Plank + Plywood + Supports |
| Column | 1.40 – 1.60 MD | 1.00 – 1.20 MD | Steel/Plywood Box or System Forms |
| Beam | 1.60 – 1.80 MD | 1.10 – 1.30 MD | Plywood + Steel Staging |

# Steel Fabrication & Fixing

| Unit | As Applicable (m³ / m² / RM / MT depending on work type) |
| --- | --- |
| Description | Execution as per approved drawings and project specifications. |
| Material Requirements | As per approved mix design / material specifications. |
| Labour Requirement | Skilled and unskilled labour as per consumption norms. |
| Machinery / Equipment | Relevant construction equipment and tools. |
| Standard Codes | IS 1200 CPWD Specifications, Relevant IRC / IS Codes |
| Quality Requirements | Compliance with tolerances, cube tests, material testing. |
| Testing Requirements | Cube test, field density test, alignment checks as applicable. |
| Safety Requirements | PPE, safe access, shoring, equipment safety compliance. |
| Remarks | Subject to site conditions and engineer approval. |

Description:

Steel fabrication and fixing form the backbone of reinforced concrete construction, ensuring structural integrity, load-bearing capacity, and durability across all project types. Efficient rebar management, accurate cutting, precise bending, and systematic tying are critical to achieving design compliance, cost efficiency, and construction timelines.

The manpower and machinery requirements for steel fabrication and fixing vary across structural elements due to differences in bar diameters, density, congestion levels, and complexity of bar shapes. Optimizing manpower and machine allocation enables faster bar processing, reduced wastage, and systematic stacking, directly impacting productivity and profitability in high-volume project environments.

The following table outlines standard manpower and machinery requirements per 1 MT of steel for key structural elements to support accurate estimation, planning, and monitoring:

Consumption Chart per 1 Ton

| Structural Element | Bar Bender (Man-days/MT) | Helper (Man-days/MT) | Cutting Machine(Hours/MT) | Bending Machine(Hours/MT) | Welding(If Applicable) |
| --- | --- | --- | --- | --- | --- |
| Rafting / Raft Slab | 6.5 – 7.5 | 4.5 – 5.5 | 1.5 – 2.0 | 1.0 – 1.2 | Rare / Edge Welding |
| Footing | 6.0 – 7.0 | 4.0 – 5.0 | 1.2 – 1.5 | 0.8 – 1.0 | Not common |
| Column | 7.0 – 8.5 | 4.5 – 5.5 | 1.5 – 1.8 | 1.2 – 1.4 | Spot Welding at laps |
| Beam | 7.5 – 9.0 | 5.0 – 6.0 | 1.8 – 2.0 | 1.3 – 1.6 | Tie Welding Optional |
| Slab | 5.5 – 6.5 | 4.0 – 5.0 | 1.0 – 1.2 | 0.8 – 1.0 | Not Required |
| Mass Fabrication | 5.0 – 6.0 | 3.5 – 4.5 | 0.8 – 1.0 | 0.6 – 0.8 | Occasional |

# Fabrication Resource Consumption

| Unit | As Applicable (m³ / m² / RM / MT depending on work type) |
| --- | --- |
| Description | Execution as per approved drawings and project specifications. |
| Material Requirements | As per approved mix design / material specifications. |
| Labour Requirement | Skilled and unskilled labour as per consumption norms. |
| Machinery / Equipment | Relevant construction equipment and tools. |
| Standard Codes | IS 456:2000, IS 1200, IS 10262, CPWD Specifications, Relevant IRC / IS Codes |
| Quality Requirements | Compliance with tolerances, cube tests, material testing. |
| Testing Requirements | Cube test, field density test, alignment checks as applicable. |
| Safety Requirements | PPE, safe access, shoring, equipment safety compliance. |
| Remarks | Subject to site conditions and engineer approval. |

Description

This matrix outlines the indicative labour, machine hours, and binding wire consumption per metric ton (MT) of reinforcement steel, segmented by bar diameter.
It serves as a baseline reference for planning, estimation, tendering, and resource allocation in rebar fabrication works (site-based or yard-based).

The values reflect practical norms under standard site/shop conditions (8-hour shifts, Fe500/Fe550 grade, medium complexity).

- Smaller diameters require higher manpower and wire consumption (more pieces per MT, tighter cages).

- Larger diameters are less labour-intensive but demand higher handling effort.

- Machine hours represent net productive time (excluding major downtime).

- Binding wire is expressed in kg/MT (~0.3–1.4% of steel weight).

Resource Consumption Matrix (Per Metric Ton)

| Bar Diameter (mm) | Bar Bender (Man-days/MT) | Helper (Man-days/MT) | Cutting Machine (Hours/MT) | Bending Machine (Hours/MT) | Binding Wire (kg/MT) |
| --- | --- | --- | --- | --- | --- |
| 6 | 9.0 – 11.0 | 6.0 – 8.0 | 1.8 – 2.2 | 1.5 – 1.9 | 10 – 14 |
| 8 | 8.0 – 10.0 | 5.5 – 7.0 | 1.6 – 2.0 | 1.3 – 1.7 | 9 – 12 |
| 10 | 7.5 – 9.0 | 5.0 – 6.5 | 1.5 – 1.8 | 1.2 – 1.6 | 8 – 11 |
| 12 | 7.0 – 8.5 | 4.5 – 6.0 | 1.3 – 1.6 | 1.1 – 1.4 | 7 – 10 |
| 16 | 6.5 – 8.0 | 4.2 – 5.5 | 1.1 – 1.4 | 1.0 – 1.3 | 6 – 9 |
| 20 | 6.0 – 7.5 | 4.0 – 5.2 | 1.0 – 1.2 | 0.9 – 1.2 | 5 – 8 |
| 25 | 5.5 – 7.0 | 3.8 – 5.0 | 0.9 – 1.1 | 0.8 – 1.1 | 4 – 7 |
| 30 | 5.0 – 6.5 | 3.5 – 4.5 | 0.8 – 1.0 | 0.7 – 1.0 | 3 – 6 |
| 35 | 4.5 – 6.0 | 3.2 – 4.2 | 0.7 – 0.9 | 0.6 – 0.9 | 3 – 5 |

# Ready Mix Concreting Work

| Unit | As Applicable (m³ / m² / RM / MT depending on work type) |
| --- | --- |
| Description | Execution as per approved drawings and project specifications. |
| Material Requirements | As per approved mix design / material specifications. |
| Labour Requirement | Skilled and unskilled labour as per consumption norms. |
| Machinery / Equipment | Relevant construction equipment and tools. |
| Standard Codes | IS 456:2000, IS 1200, IS 10262, CPWD Specifications, Relevant IRC / IS Codes |
| Quality Requirements | Compliance with tolerances, cube tests, material testing. |
| Testing Requirements | Cube test, field density test, alignment checks as applicable. |
| Safety Requirements | PPE, safe access, shoring, equipment safety compliance. |
| Remarks | Subject to site conditions and engineer approval. |

Description:
Ready-Mix Concrete (RMC) work involves the placement, compaction, and finishing of pre-mixed concrete delivered in transit mixers, ensuring consistent quality, speed, and reduced site-level batching and mixing activities. It is adopted for low quantity concreting (columns, footings), mass quantity concreting (beams, slabs), and high-volume concreting (rafts, podium slabs, industrial floors) as per project needs.

RMC enhances construction efficiency by minimizing wastage, controlling water-cement ratios, and ensuring quality compliance with IS standards and project-specific mix designs. Execution requires planning for manpower, equipment (pumps, vibrators, screeds), site logistics, and QC monitoring (slump, cube testing) to align with timelines and structural durability.

Consumption Chart per 1 m³

RMC Laying Only – Low Quantity Concreting

| Component | M5 | M7.5 | M10 | M15 | M20 | M25 | M30 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cement (in RMC) | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| Sand + Jelly (Premix) | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| Water (Premixed) | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| Mason (Laying/Finishing) | 0.20 MD | 0.22 MD | 0.24 MD | 0.28 MD | 0.30 MD | 0.32 MD | 0.35 MD |
| Helper | 0.15 MD | 0.18 MD | 0.20 MD | 0.22 MD | 0.25 MD | 0.26 MD | 0.28 MD |
| Concrete Vibrator | 0.05 Hr | 0.06 Hr | 0.08 Hr | 0.10 Hr | 0.12 Hr | 0.15 Hr | 0.17 Hr |
| Transit Mixer | 0.10 Hr | 0.10 Hr | 0.10 Hr | 0.10 Hr | 0.10 Hr | 0.10 Hr | 0.10 Hr |
| Concrete Pump (if used) | Optional | Optional | 0.10 Hr | 0.12 Hr | 0.15 Hr | 0.18 Hr | 0.20 Hr |
| Shuttering, Steel | Not Included across all grades |  |  |  |  |  |  |

Consumption Chart per 1 m³

RMC Laying Only – Mass Quantity Concreting

| Component | M5 | M7.5 | M10 | M15 | M20 | M25 | M30 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cement / RMC Mix | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| Sand + Jelly (Premix) | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| Water (Premixed) | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| Mason (Spreading/Finishing) | 0.15 MD | 0.16 MD | 0.18 MD | 0.20 MD | 0.22 MD | 0.24 MD | 0.26 MD |
| Helper | 0.12 MD | 0.14 MD | 0.15 MD | 0.16 MD | 0.18 MD | 0.20 MD | 0.22 MD |
| Vibrator (Compaction) | Not Used | 0.04 Hr | 0.05 Hr | 0.08 Hr | 0.10 Hr | 0.12 Hr | 0.15 Hr |
| Transit Mixer | 0.10 Hr | 0.10 Hr | 0.10 Hr | 0.10 Hr | 0.10 Hr | 0.10 Hr | 0.10 Hr |
| Concrete Pump (if used) | Optional | Optional | 0.10 Hr | 0.12 Hr | 0.14 Hr | 0.16 Hr | 0.18 Hr |
| Formwork & Steel | Not Included across all grades |  |  |  |  |  |  |

Consumption Chart per 1 m³

RMC Laying Only – High Volume Mass Quantity Concreting

| Component | M5 | M7.5 | M10 | M15 | M20 | M25 | M30 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cement / RMC Mix | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| Sand + Jelly (Premixed) | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| Water (Premixed) | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| Mason (Finishing only) | 0.08 MD | 0.10 MD | 0.12 MD | 0.14 MD | 0.16 MD | 0.18 MD | 0.20 MD |
| Helper | 0.06 MD | 0.08 MD | 0.10 MD | 0.12 MD | 0.14 MD | 0.15 MD | 0.16 MD |
| Vibrator Operator | 0.03 Hr | 0.04 Hr | 0.06 Hr | 0.08 Hr | 0.10 Hr | 0.12 Hr | 0.14 Hr |
| Transit Mixer (7 CUM) | 0.14 Trip across all grades |  |  |  |  |  |  |
| Concrete Pump | 0.08 Hr | 0.10 Hr | 0.12 Hr | 0.14 Hr | 0.16 Hr | 0.18 Hr | 0.20 Hr |
| Finishing Equipment | Bull Float / Screed Board / Screed Vibrator (as required) |  |  |  |  |  |  |
| QC (Cube Casting) | 1 Set per 15–20 m³ poured across all grades |  |  |  |  |  |  |

# Concrete Work - Site Mix - Structure Element

| Unit | As Applicable (m³ / m² / RM / MT depending on work type) |
| --- | --- |
| Description | Execution as per approved drawings and project specifications. |
| Material Requirements | As per approved mix design / material specifications. |
| Labour Requirement | Skilled and unskilled labour as per consumption norms. |
| Machinery / Equipment | Relevant construction equipment and tools. |
| Standard Codes | IS 456:2000, IS 10262, CPWD Specifications, Relevant IRC / IS Codes |
| Quality Requirements | Compliance with tolerances, cube tests, material testing. |
| Testing Requirements | Cube test, field density test, alignment checks as applicable. |
| Safety Requirements | PPE, safe access, shoring, equipment safety compliance. |
| Remarks | Subject to site conditions and engineer approval. |

Description:
Concrete work using site mix involves accurate batching, mixing, placing, compacting, and finishing of concrete directly at the project site using cement, sand, aggregates, and water in specified proportions to achieve the required grade (M20, M25, M30). This method is practical for projects requiring flexible pour schedules, control over workability, and cost optimization.

Key operational considerations include:

- Material control: Storing and managing cement, sand, and aggregates to prevent contamination or moisture ingress.

- Proportioning: Using calibrated measures for cement bags, volume boxes for sand/aggregates, and measured water for consistent quality.

- Mixing: Using mechanical mixers to ensure homogeneity.

- Placement: Timely pouring to avoid cold joints, maintaining workability through planned sequencing.

- Compaction: Using vibrators for effective removal of air voids to enhance density and strength.

- Finishing: Skilled masons ensure proper levelling and surface finish.

- Curing: Ensuring post-pour curing to achieve designed strength and durability.

Site mix concreting is ideal where project scale, logistics, and customization of concrete properties warrant on-site preparation, supporting quality-centric execution aligned with IS 456:2000 and IS 10262 standards.

Consumption Chart Per 1 m³

Concrete Work – M30 Grade (Per 1 m³) by Structural Element

| Component | Footing | Pedestal | Column | Plinth Beam |
| --- | --- | --- | --- | --- |
| Cement (OPC 53) | 12.5 Bags | 12.5 Bags | 12.5 Bags | 12.5 Bags |
| Sand | 0.39 m³ | 0.39 m³ | 0.39 m³ | 0.39 m³ |
| Jelly (Aggregate) | 0.76 m³ (20 + 10 mm) | 0.76 m³ (20 + 10 mm) | 0.76 m³ (20 + 10 mm) | 0.76 m³ (20 + 10 mm) |
| Water | 190–210 L | 190–210 L | 190–210 L | 190–210 L |
| Mason | 0.50 MD | 0.60 MD | 0.70 MD | 0.65 MD |
| Helper | 0.75 MD | 0.80 MD | 0.90 MD | 0.85 MD |
| Carpenter (Shuttering) | 0.40 MD | 0.45 MD | 0.60 MD | 0.55 MD |
| Bar Bender | 0.35 MD | 0.40 MD | 0.45 MD | 0.40 MD |
| Mixing Machine | 0.15 Hr | 0.15 Hr | 0.15 Hr | 0.15 Hr |
| Vibrator | 0.15 Hr | 0.17 Hr | 0.20 Hr | 0.17 Hr |
| Shuttering Material | Tight Plywood + Props | Plywood + Steel Plates | Steel / Precision Formwork | Tight Plywood + Framing |

Concrete Work – M25 Grade (Per 1 m³) by Structural Element

| Component | Footing | Pedestal | Column | Plinth Beam |
| --- | --- | --- | --- | --- |
| Cement (OPC 53) | 11.5 Bags | 11.5 Bags | 11.5 Bags | 11.5 Bags |
| Sand | 0.40 m³ | 0.40 m³ | 0.40 m³ | 0.40 m³ |
| Jelly (Aggregate) | 0.77 m³ (20 + 10 mm) | 0.77 m³ (20 + 10 mm) | 0.77 m³ (20 + 10 mm) | 0.77 m³ (20 + 10 mm) |
| Water | 200–210 L | 200–210 L | 200–210 L | 200–210 L |
| Mason | 0.45 MD | 0.55 MD | 0.65 MD | 0.60 MD |
| Helper | 0.70 MD | 0.75 MD | 0.85 MD | 0.80 MD |
| Carpenter (Shuttering) | 0.35 MD | 0.40 MD | 0.55 MD | 0.50 MD |
| Bar Bender | 0.30 MD | 0.35 MD | 0.40 MD | 0.35 MD |
| Mixing Machine | 0.15 Hr | 0.15 Hr | 0.15 Hr | 0.15 Hr |
| Vibrator | 0.12 Hr | 0.15 Hr | 0.18 Hr | 0.15 Hr |
| Shuttering Material | Tight Plywood + Planks | Plywood + Framing | Steel / Modular Formwork | Tight Plywood + Props |

Concrete Work – M20 Grade (Per 1 m³) by Structural Element

| Component | Footing | Pedestal | Column | Plinth Beam |
| --- | --- | --- | --- | --- |
| Cement (OPC 53) | 10.5 Bags | 10.5 Bags | 10.5 Bags | 10.5 Bags |
| Sand | 0.42 m³ | 0.42 m³ | 0.42 m³ | 0.42 m³ |
| Jelly (Aggregate) | 0.78 m³ (20 + 10 mm) | 0.78 m³ (20 + 10 mm) | 0.78 m³ (20 + 10 mm) | 0.78 m³ (20 + 10 mm) |
| Water | 210–230 L | 210–230 L | 210–230 L | 210–230 L |
| Mason | 0.40 MD | 0.50 MD | 0.60 MD | 0.55 MD |
| Helper | 0.65 MD | 0.70 MD | 0.80 MD | 0.75 MD |
| Carpenter (Shuttering) | 0.30 MD | 0.35 MD | 0.50 MD | 0.45 MD |
| Bar Bender | 0.25 MD | 0.30 MD | 0.35 MD | 0.30 MD |
| Mixing Machine | 0.15 Hr | 0.15 Hr | 0.15 Hr | 0.15 Hr |
| Vibrator | 0.10 Hr | 0.12 Hr | 0.15 Hr | 0.12 Hr |
| Shuttering Material | Basic Plywood + Planks | Plywood + Tight Framing | Steel / Heavy Plywood | Tight Plank + Props |

# Concrete Work - Site Mix -   Super Structure Element

| Unit | As Applicable (m³ / m² / RM / MT depending on work type) |
| --- | --- |
| Description | Execution as per approved drawings and project specifications. |
| Material Requirements | As per approved mix design / material specifications. |
| Labour Requirement | Skilled and unskilled labour as per consumption norms. |
| Machinery / Equipment | Relevant construction equipment and tools. |
| Standard Codes | IS 456:2000, IS 1200 CPWD Specifications, Relevant IRC / IS Codes |
| Quality Requirements | Compliance with tolerances, cube tests, material testing. |
| Testing Requirements | Cube test, field density test, alignment checks as applicable. |
| Safety Requirements | PPE, safe access, shoring, equipment safety compliance. |
| Remarks | Subject to site conditions and engineer approval. |

Description:
Concrete work for super structure elements (RCC walls, staircases, beams, slabs) using site mix involves systematic batching, mixing, placement, compaction, and finishing to achieve structurally sound, durable, and high-quality components in M20, M25, and M30 grades.

This approach ensures precise material control, consistent workability, and site adaptability, addressing diverse project requirements while aligning with IS 456:2000 and good construction practices. It includes:

- Material management: Using OPC 53 cement, washed sand, and well-graded aggregates (20 mm + 10 mm).

- Site-controlled batching and mixing: Utilizing calibrated volumetric measurement and mechanical mixers for uniformity.

- Skilled placement and compaction: Executed by trained masons with mechanical vibration to avoid honeycombing.

- Quality formwork: Utilizing steel or plywood systems as per element type to maintain dimensional accuracy and surface finish.

- Curing: Ensuring appropriate curing methods to achieve design strength and durability.

This methodology is suitable for mid-rise and high-rise constructions, providing quality assurance, cost optimization, and execution flexibility in large-scale super structure activities.

Consumption Chart Per 1 m³

Concrete Work – M30 Grade (Per 1 m³) by Super Structure Element

| Component | RCC Wall | Staircase | Beam | Slab |
| --- | --- | --- | --- | --- |
| Cement (OPC 53) | 12.5 Bags | 12.5 Bags | 12.5 Bags | 12.5 Bags |
| Sand | 0.39 m³ | 0.39 m³ | 0.39 m³ | 0.39 m³ |
| Jelly (Aggregate) | 0.76 m³ (20+10 mm) | 0.76 m³ (20+10 mm) | 0.76 m³ (20+10 mm) | 0.76 m³ (20+10 mm) |
| Water | 190–210 L | 190–210 L | 190–210 L | 190–210 L |
| Mason | 0.60 MD | 0.70 MD | 0.75 MD | 0.65 MD |
| Helper | 0.85 MD | 0.90 MD | 1.00 MD | 0.90 MD |
| Carpenter (Shuttering) | 0.50 MD | 0.55 MD | 0.65 MD | 0.60 MD |
| Bar Bender | 0.40 MD | 0.45 MD | 0.50 MD | 0.40 MD |
| Mixing Machine | 0.15 Hr | 0.15 Hr | 0.15 Hr | 0.15 Hr |
| Vibrator | 0.18 Hr | 0.20 Hr | 0.22 Hr | 0.20 Hr |
| Shuttering Material | Steel/Plywood Panel Wall Form | Custom Plywood + Angles | Steel/Ply Box + Supports | Centring + Beam Slab Panels |

Concrete Work – M25 Grade (Per 1 m³) by Super Structure Element

| Component | RCC Wall | Staircase | Beam | Slab |
| --- | --- | --- | --- | --- |
| Cement (OPC 53) | 11.5 Bags | 11.5 Bags | 11.5 Bags | 11.5 Bags |
| Sand | 0.40 m³ | 0.40 m³ | 0.40 m³ | 0.40 m³ |
| Jelly (Aggregate) | 0.77 m³ (20+10 mm) | 0.77 m³ (20+10 mm) | 0.77 m³ (20+10 mm) | 0.77 m³ (20+10 mm) |
| Water | 200–210 L | 200–210 L | 200–210 L | 200–210 L |
| Mason | 0.55 MD | 0.65 MD | 0.70 MD | 0.60 MD |
| Helper | 0.80 MD | 0.85 MD | 0.95 MD | 0.85 MD |
| Carpenter (Shuttering) | 0.45 MD | 0.50 MD | 0.60 MD | 0.55 MD |
| Bar Bender | 0.35 MD | 0.40 MD | 0.45 MD | 0.35 MD |
| Mixing Machine | 0.15 Hr | 0.15 Hr | 0.15 Hr | 0.15 Hr |
| Vibrator | 0.15 Hr | 0.18 Hr | 0.20 Hr | 0.18 Hr |
| Shuttering Material | Plywood Panel + Supports | Custom Plywood + Angles | Steel/Ply Box + Supports | Slab Centring with Props |

Concrete Work – M20 Grade (Per 1 m³) by Super Structure Element

| Component | RCC Wall | Staircase | Beam | Slab |
| --- | --- | --- | --- | --- |
| Cement (OPC 53) | 10.5 Bags | 10.5 Bags | 10.5 Bags | 10.5 Bags |
| Sand | 0.42 m³ | 0.42 m³ | 0.42 m³ | 0.42 m³ |
| Jelly (Aggregate) | 0.78 m³ (20+10 mm) | 0.78 m³ (20+10 mm) | 0.78 m³ (20+10 mm) | 0.78 m³ (20+10 mm) |
| Water | 210–230 L | 210–230 L | 210–230 L | 210–230 L |
| Mason | 0.50 MD | 0.60 MD | 0.65 MD | 0.55 MD |
| Helper | 0.75 MD | 0.80 MD | 0.90 MD | 0.80 MD |
| Carpenter (Shuttering) | 0.40 MD | 0.45 MD | 0.55 MD | 0.50 MD |
| Bar Bender | 0.30 MD | 0.35 MD | 0.40 MD | 0.30 MD |
| Mixing Machine | 0.15 Hr | 0.15 Hr | 0.15 Hr | 0.15 Hr |
| Vibrator | 0.12 Hr | 0.15 Hr | 0.18 Hr | 0.15 Hr |
| Shuttering Material | Plywood Panel + Bracing | Custom Plywood + Mild Steel | Steel/Plywood + Supports | MS Centring + Horizontal Props |

# Concrete Work – Site Mix -

Description:
Super structure slab work using M20 grade site mix concrete requires precise control over material quantities, batching, mixing, placement, compaction, and curing to achieve high-quality, durable slabs. The material consumption varies based on slab thickness (150 mm, 100 mm, 75 mm), with integrated planning for reinforcement, formwork, and mechanical vibration ensuring structural performance while optimizing resources.

This systematic approach ensures:

✅ Accurate material estimation for project costing and planning
✅ Efficient site execution with minimal wastage
✅ Consistency in slab quality across varying thicknesses

Consumption Chart Per  1m²

| Component | 150 mm Slab | 100 mm Slab | 75 mm Slab |
| --- | --- | --- | --- |
| Concrete Volume | 0.15 m³ | 0.10 m³ | 0.075 m³ |
| Cement (OPC 53) | 1.58 Bags | 1.05 Bags | 0.79 Bags |
| Sand | 0.063 m³ | 0.042 m³ | 0.031 m³ |
| Jelly (Aggregate) | 0.117 m³ | 0.078 m³ | 0.058 m³ |
| Water | 31 – 34 L | 21 – 23 L | 15 – 18 L |
| Mason | 0.09 Man-day | 0.06 Man-day | 0.045 Man-day |
| Helper | 0.12 Man-day | 0.08 Man-day | 0.06 Man-day |
| Carpenter (Shuttering) | 0.07 Man-day | 0.05 Man-day | 0.035 Man-day |
| Bar Bender | 0.05 Man-day | 0.035 Man-day | 0.025 Man-day |
| Mixing Machine | 0.022 Hr | 0.015 Hr | 0.011 Hr |
| Vibrator | 0.018 Hr | 0.012 Hr | 0.009 Hr |
| Shuttering Material | MS Props + Plywood / Centering Sheets | MS Props + Plywood / Centering Sheets | MS Props + Plywood / Centering Sheets |

# Masonry Work

Laterite Stone Masonry Work (Per 1 m² Wall)

Description:
Laterite stone masonry is a durable, sustainable, and regionally appropriate wall construction technique used in residential, institutional, and light commercial structures, especially across coastal and plateau zones. It utilizes locally sourced laterite blocks with cement mortar of varying mixes, depending on strength and budget requirements. This chart standardizes resource estimation, ensuring clarity in planning, procurement, and rate analysis for executing laterite masonry efficiently on site.

Consumption Chart Per 1m²

Laterite Stone Masonry (Per 1 m² of 9" Wall)

| Component | 1:3 CM | 1:4 CM | 1:6 CM |
| --- | --- | --- | --- |
| Wall Thickness | 9 inches (0.225 m) | 9 inches (0.225 m) | 9 inches (0.225 m) |
| Stone Size | 9" x 13" x 7" | 9" x 13" x 7" | 9" x 13" x 7" |
| No. of Stones | ~16 per m² | ~16 per m² | ~16 per m² |
| Cement | ~0.45 bags (~22 kg) | ~0.34 bags (~17 kg) | ~0.23 bags (~11.5 kg) |
| Sand | 0.068 m³ | 0.068 m³ | 0.068 m³ |
| Water | 7 – 9 L | 6 – 8 L | 5 – 6 L |
| Mason | 0.15 – 0.16 MD | 0.14 – 0.15 MD | 0.12 – 0.13 MD |
| Helper | 0.12 – 0.13 MD | 0.11 – 0.12 MD | 0.10 – 0.11 MD |

Laterite Stone Masonry (Per 1 m² of 8" Wall)

| Component | 1:3 CM | 1:4 CM | 1:6 CM |
| --- | --- | --- | --- |
| Wall Thickness | 8 inches (0.20 m) | 8 inches (0.20 m) | 8 inches (0.20 m) |
| Stone Size | 9" x 13" x 7" | 9" x 13" x 7" | 9" x 13" x 7" |
| No. of Stones | ~15 per m² | ~15 per m² | ~15 per m² |
| Cement | ~0.40 bags (~20 kg) | ~0.30 bags (~15 kg) | ~0.20 bags (~10 kg) |
| Sand | 0.060 m³ | 0.060 m³ | 0.060 m³ |
| Water | 6 – 8 L | 5 – 7 L | 4 – 5 L |
| Mason | 0.14 – 0.15 MD | 0.13 – 0.14 MD | 0.11 – 0.12 MD |
| Helper | 0.11 – 0.12 MD | 0.10 – 0.11 MD | 0.09 – 0.10 MD |

Laterite Stone Masonry (Per 1 m² of 6" Wall)

| Component | 1:3 CM | 1:4 CM | 1:6 CM |
| --- | --- | --- | --- |
| Wall Thickness | 6 inches (0.15 m) | 6 inches (0.15 m) | 6 inches (0.15 m) |
| Stone Size | 9" x 13" x 7" | 9" x 13" x 7" | 9" x 13" x 7" |
| No. of Stones | ~11 per m² | ~11 per m² | ~11 per m² |
| Cement | ~0.30 bags (~15 kg) | ~0.22 bags (~11 kg) | ~0.15 bags (~7.5 kg) |
| Sand | 0.045 m³ | 0.045 m³ | 0.045 m³ |
| Water | 5 – 6 L | 4 – 5 L | 3 – 4 L |
| Mason | 0.13 – 0.14 MD | 0.12 – 0.13 MD | 0.10 – 0.11 MD |
| Helper | 0.10 – 0.11 MD | 0.09 – 0.10 MD | 0.08 – 0.09 MD |

Consumption Chart Per 1m³

Laterite Stone Per 1 m³ of Masonry

| Component | 1:4 Mix | 1:6 Mix |
| --- | --- | --- |
| Stone Size | 9" x 13" x 7" | 9" x 13" x 7" |
| Stones per m³ | ~70 – 72 stones | ~70 – 72 stones |
| Cement | 1.5 bags | 1.0 bag |
| Sand | 0.30 m³ | 0.30 m³ |
| Water | 25 – 30 L | 20 – 25 L |
| Mason | 0.60 – 0.65 Man-day | 0.55 – 0.60 Man-day |
| Helper | 0.50 – 0.55 Man-day | 0.45 – 0.50 Man-day |

# Cement Block Masonry

| Unit of Measurement | m³ |
| --- | --- |
| Block Size | 400×200×200 mm |
| Blocks per m³ | 125–135 Nos |
| Mortar Mix | 1:6 Cement Mortar |
| Mortar Volume | 0.30–0.35 m³ |
| Cement | 1.6–1.9 bags |
| Sand | 0.30–0.35 m³ |
| Water | 180–200 Litres |
| Mason Labour | 0.80–1.00 Man-day |
| Helper Labour | 0.60–0.80 Man-day |
| Applicable Standards | IS 2185 (Part 1), IS 2212, IS 3495, IS 1200 Part 3 |
| Joint Thickness | 10 mm |

Cement Block Masonry Work (Per 1 m² Wall)

Description:
Cement block masonry is widely used for robust, efficient wall construction in residential and commercial structures, ensuring faster progress while maintaining durability. This chart standardizes material, labor, and water consumption for CM 1:3, 1:4, and 1:5 mixes across 8", 6", and 4" wall thicknesses for precise BOQ, estimation, and planning within your workflows.

Consumption Chart Per 1m²

Cement Block Masonry (Per 1 m², 8" Wall)

| Component | 1:3 Mortar Mix | 1:4 Mortar Mix | 1:5 Mortar Mix |
| --- | --- | --- | --- |
| No. of Blocks | 13 blocks (8"x8"x16") | 13 blocks (8"x8"x16") | 13 blocks (8"x8"x16") |
| Cement | ~0.36 bags (~18 kg) | ~0.28 bags (~14 kg) | ~0.22 bags (~11 kg) |
| Sand | ~0.060 m³ | ~0.055 m³ | ~0.055 m³ |
| Water | 4.5 – 5.5 L | 4 – 5 L | 3.5 – 4.5 L |
| Mason | 0.15 – 0.16 Man-day | 0.14 – 0.15 Man-day | 0.13 – 0.14 Man-day |
| Helper | 0.12 – 0.13 Man-day | 0.11 – 0.12 Man-day | 0.10 – 0.11 Man-day |

Cement Block Masonry (Per 1 m², 6" Wall)

| Component | 1:3 Mortar Mix | 1:4 Mortar Mix | 1:5 Mortar Mix |
| --- | --- | --- | --- |
| No. of Blocks | 10 blocks (6"x8"x16") | 10 blocks (6"x8"x16") | 10 blocks (6"x8"x16") |
| Cement | ~0.28 bags (~14 kg) | ~0.23 bags (~11.5 kg) | ~0.18 bags (~9 kg) |
| Sand | ~0.050 m³ | ~0.045 m³ | ~0.045 m³ |
| Water | 4 – 5 L | 3.5 – 4.5 L | 3 – 4 L |
| Mason | 0.14 – 0.15 Man-day | 0.13 – 0.14 Man-day | 0.12 – 0.13 Man-day |
| Helper | 0.11 – 0.12 Man-day | 0.10 – 0.11 Man-day | 0.09 – 0.10 Man-day |

Cement Block Masonry (Per 1 m², 4" Wall)

| Component | 1:3 Mortar Mix | 1:4 Mortar Mix | 1:5 Mortar Mix |
| --- | --- | --- | --- |
| No. of Blocks | 7 blocks (4"x8"x16") | 7 blocks (4"x8"x16") | 7 blocks (4"x8"x16") |
| Cement | ~0.24 bags (~12 kg) | ~0.20 bags (~10 kg) | ~0.15 bags (~7.5 kg) |
| Sand | ~0.045 m³ | ~0.040 m³ | ~0.040 m³ |
| Water | 3.5 – 4.5 L | 3 – 4 L | 2.5 – 3.5 L |
| Mason | 0.13 – 0.14 Man-day | 0.12 – 0.13 Man-day | 0.11 – 0.12 Man-day |
| Helper | 0.10 – 0.11 Man-day | 0.09 – 0.10 Man-day | 0.08 – 0.09 Man-day |

# ACC Ash Block Masonry

Ash Block Masonry Work (Per 1 m² Wall)

| Unit of Measurement | m³ |
| --- | --- |
| Block Size | 600×200×200 mm |
| Blocks per m³ | 27–30 Nos |
| Adhesive Consumption | 8–10 kg |
| Cement (if CM used) | 1.2–1.5 bags |
| Sand (if CM used) | 0.25–0.30 m³ |
| Water | 40–50 Litres |
| Mason Labour | 0.70–0.85 Man-day |
| Helper Labour | 0.50–0.65 Man-day |
| Applicable Standards | IS 2185 (Part 3), IS 6041, IS 1200 Part 3 |
| Joint Thickness | 3–5 mm |

Description:
AAC (Autoclaved Aerated Concrete) block masonry with factory-made adhesive enables high-precision construction with reduced dead loads, thermal insulation, and faster work cycles. The below chart standardizes material, adhesive, water consumption, and labor requirements for 8", 6", and 4" walls, streamlining your BOQ estimation and resource planning.

Consumption Chart Per 1m²

| Component | 8" Wall (0.20 m) | 6" Wall (0.15 m) | 4" Wall (0.10 m) |
| --- | --- | --- | --- |
| Block Size | 8" x 8" x 24" | 6" x 8" x 24" | 4" x 8" x 24" |
| No. of Blocks/m² | 9 blocks | 9 blocks | 9 blocks |
| Adhesive (Ready Mix) | 5 – 6 kg/m² | 3.8 – 4.5 kg/m² | 2.5 – 3.5 kg/m² |
| Water for Adhesive | 0.5 – 0.7 L/m² | 0.4 – 0.5 L/m² | 0.3 – 0.4 L/m² |
| Mason | 0.12 – 0.13 Man-day | 0.11 – 0.12 Man-day | 0.11 – 0.12 Man-day |
| Helper | 0.10 – 0.11 Man-day | 0.09 – 0.10 Man-day | 0.09 – 0.10 Man-day |

# Plastering Work – Internal

| Unit of Measurement | m² |
| --- | --- |
| Mortar Mix | 1:6 Cement Mortar |
| Cement | 0.18–0.22 bags |
| Sand | 0.015–0.018 m³ |
| Water | 4–5 Litres |
| Mason Labour | 0.15–0.18 Man-day |
| Helper Labour | 0.12–0.15 Man-day |
| Thickness | 12 mm |
| Applicable Standards | IS 1661, IS 2402, IS 1200 Part 12 |
| Tolerance | ±3 mm |

Internal Ceiling Plastering – 12 mm Thick (Per 1 m²)

Description:
Internal ceiling plastering with 12 mm thickness ensures a smooth, crack-resistant surface for receiving paint or texture finishes, with accurate control on consumption of cement, sand, water, and manpower. The chart below outlines the material and labor requirements for 1:3, 1:4, and 1:6 mix ratios aligned with standard site execution practices.

Consumption Chart Per 1m²

| Component | 1:3 Mix Ratio | 1:4 Mix Ratio | 1:6 Mix Ratio |
| --- | --- | --- | --- |
| Cement (OPC / PPC) | 0.28 bag (14 kg) | 0.23 bag (11.5 kg) | 0.16 bag (8 kg) |
| Sand (River Sand) | 0.036 m³ | 0.042 m³ | 0.042 m³ |
| Water | 5 – 6.5 L | 5 – 6 L | 5 – 6 L |
| Mason (Plasterer) | 0.25 Man-day | 0.22 – 0.24 Man-day | 0.20 – 0.22 Man-day |
| Helper | 0.16 Man-day | 0.14 – 0.15 Man-day | 0.13 – 0.14 Man-day |

Internal Wall Plastering – 12 mm Thick (Per 1 m²)

Description:
Internal wall plastering with 12 mm thickness is executed to provide an even, durable, and aesthetically pleasing surface for paint and finishes, while enhancing moisture resistance. Accurate measurement of cement, sand, water, and labor ensures cost control and quality in residential, commercial, and institutional projects.

Consumption Chart Per 1m²

| Component | 1:3 Mix Ratio | 1:4 Mix Ratio | 1:6 Mix Ratio |
| --- | --- | --- | --- |
| Cement (OPC / PPC) | 0.28 Bag (14 kg) | 0.22 Bag (11 kg) | 0.15 Bag (7.5 kg) |
| Sand (River Sand) | 0.036 m³ | 0.042 m³ | 0.042 m³ |
| Water | 5 – 6.5 L | 4 – 5 L | 4 – 5 L |
| Mason (Plasterer) | 0.22 – 0.25 Man-day | 0.18 – 0.20 Man-day | 0.17 – 0.19 Man-day |
| Helper | 0.14 – 0.16 Man-day | 0.12 – 0.14 Man-day | 0.10 – 0.12 Man-day |

# Plastering Work – External

| Unit of Measurement | m² |
| --- | --- |
| Mortar Mix | 1:4 Cement Mortar |
| Cement | 0.25–0.30 bags |
| Sand | 0.020–0.025 m³ |
| Water | 5–6 Litres |
| Mason Labour | 0.18–0.22 Man-day |
| Helper Labour | 0.14–0.18 Man-day |
| Thickness | 12–20 mm |
| Applicable Standards | IS 1661, IS 2402 |
| Curing Period | 7 days minimum |

External Ceiling Plastering – 12 mm Thick (Per 1 m²)

Description:
External ceiling plastering of 12 mm thickness with a single coat is applied to enhance weather resistance, protect structural elements, and prepare surfaces for paint or textured finishes in balconies, sunshades, and soffits. Material and labor consumption planning is essential for quality finish and cost management across high-rise, villa, and commercial projects.

Consumption Chart Per 1m²

| Component | 1:3 Mix Ratio | 1:4 Mix Ratio | 1:6 Mix Ratio |
| --- | --- | --- | --- |
| Cement (OPC / PPC) | 0.28 Bag (14 kg) | 0.22 Bag (11 kg) | 0.15 Bag (7.5 kg) |
| Sand (River Sand) | 0.036 m³ | 0.042 m³ | 0.042 m³ |
| Water | 5 – 6.5 L | 4 – 5 L | 4 – 5 L |
| Mason (Plasterer) | 0.24 – 0.27 Man-day | 0.21 – 0.24 Man-day | 0.19 – 0.22 Man-day |
| Helper | 0.16 – 0.18 Man-day | 0.14 – 0.16 Man-day | 0.13 – 0.15 Man-day |

External Ceiling Plastering – 12 mm Thick (Single Coat) per 1 m²

Description:
External ceiling plastering with 12 mm thickness in a single coat is applied to soffits, balcony ceilings, and exposed projections to enhance durability against weather and improve aesthetics before painting or waterproofing. It ensures a level, crack-resistant surface, reducing seepage risks while maintaining structural protection. Selection of the mix ratio (1:3, 1:4, 1:6) impacts cement consumption and finish quality, which should align with project specifications and climatic conditions for sustainable cost management.

Consumption Chart Per 1m²

| Component | 1:3 Mix Ratio | 1:4 Mix Ratio | 1:6 Mix Ratio |
| --- | --- | --- | --- |
| Cement (OPC / PPC) | 0.28 Bag (14 kg) | 0.22 Bag (11 kg) | 0.15 Bag (7.5 kg) |
| Sand (River Sand) | 0.036 m³ | 0.042 m³ | 0.042 m³ |
| Water | 5 – 6.5 L | 4 – 5 L | 4 – 5 L |
| Mason (Plasterer) | 0.24 – 0.27 Man-day | 0.21 – 0.24 Man-day | 0.19 – 0.22 Man-day |
| Helper | 0.16 – 0.18 Man-day | 0.14 – 0.16 Man-day | 0.13 – 0.15 Man-day |

External Wall Plastering – 18 mm (Double Coat) per 1 m²   (12mm + 6mm Plaster)

Description:
External wall plastering at 18 mm thickness in double coat (12 mm base coat + 6 mm finishing coat) is executed to protect external masonry against weathering, enhance aesthetics, and prepare surfaces for painting or waterproofing. The first coat (scratch coat) ensures bonding and surface levelling, while the second coat refines the finish, ensuring crack resistance and improved durability. Appropriate mix selection (1:3, 1:4, 1:6) balances surface finish quality, water tightness, and project cost-efficiency, aligning with modern facade protection strategies in premium residential and commercial developments.

Consumption Chart Per 1m²

| Component | 1:3 Mix Ratio | 1:4 Mix Ratio | 1:6 Mix Ratio |
| --- | --- | --- | --- |
| Cement (OPC / PPC) | 0.45 Bag (22.5 kg) | 0.36 Bag (18 kg) | 0.25 Bag (12.5 kg) |
| Sand (River Sand) | 0.065 m³ | 0.075 m³ | 0.075 m³ |
| Water | 8 – 10 L | 7 – 9 L | 7 – 9 L |
| Mason (Plasterer) | 0.32 – 0.35 Man-day | 0.28 – 0.30 Man-day | 0.25 – 0.28 Man-day |
| Helper | 0.20 – 0.22 Man-day | 0.18 – 0.20 Man-day | 0.16 – 0.18 Man-day |

# Waterproofing Work

| Unit of Measurement | m² |
| --- | --- |
| Type | Cementitious / Liquid Membrane |
| Material Consumption | 1.5–2.0 kg/m² |
| Primer | 0.20 kg/m² |
| Labour | 0.12–0.18 Man-day |
| Applicable Standards | IS 3067 |
| Testing | 24–48 hours ponding test |

Waterproof Plastering – Material & Manpower Consumption per 1 m² (Internal Work)

Description:
Internal waterproof plastering safeguards interiors against moisture ingress, seepage, and dampness while providing a smooth finish ready for paint or cladding. Advanced methods like polymer, crystalline, fibre-reinforced, and integral waterproof plastering enhance impermeability and surface durability. These methods align with modern sustainable construction practices, ensuring longevity in high-humidity areas such as bathrooms, basements, kitchens, and utility ducts. Selection of plaster type depends on project budget, criticality of waterproofing, and finish quality required in premium residential and commercial projects.

Consumption Chart Per 1m²

| Plaster Type | Mix Ratio | Thickness | Cement | Sand | Additive / Chemical | Water | Mason | Helper |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cement Polymer Plaster | 1:4 | 15 mm | 0.28 bag (14 kg) | 0.045 m³ | SBR Polymer ~150 ml | 5 – 6 L | 0.22 – 0.25 MD | 0.15 – 0.17 MD |
| Integral Waterproof Plaster | 1:5 | 18 mm | 0.30 bag (15 kg) | 0.052 m³ | Integral Compound ~150 g | 6 – 7 L | 0.24 – 0.26 MD | 0.16 – 0.18 MD |
| Crystalline Waterproof Plaster | 1:4 | 15 mm | 0.28 bag (14 kg) | 0.045 m³ | Crystalline Powder ~250 g | 5 – 6 L | 0.22 – 0.25 MD | 0.15 – 0.17 MD |
| Fibre-Reinforced WP Plaster | 1:4 | 20 mm | 0.36 bag (18 kg) | 0.060 m³ | Polymer 150 ml + Fibre Mesh (0.9 m²) | 6 – 7 L | 0.26 – 0.28 MD | 0.18 – 0.20 MD |
| Bitumen Layer over Cement Plaster | 1:3 | 20 mm + coat | 0.42 bag (21 kg) | 0.065 m³ | Bitumen Coating ~1.25 kg | 7 – 8 L | 0.28 – 0.30 MD | 0.18 – 0.20 MD |
| Polymer Mortar Screed Plaster | 1:3 | 20 mm | 0.42 bag (21 kg) | 0.065 m³ | SBR Polymer ~250 ml | 7 – 8 L | 0.28 – 0.30 MD | 0.18 – 0.20 MD |
| Waterproof Putty Finish (Top Coat) | Ready Mix | 3 mm (only) | — | — | Acrylic Putty ~1.2–1.5 kg | 0.3 L | 0.12 – 0.15 MD | 0.08 – 0.10 MD |

Waterproof Plastering – Material & Manpower Consumption per 1 m² (External Work)

Description:
External waterproof plastering is essential for protecting building façades from heavy rainfall, wind-driven moisture, and capillary water ingress while providing a durable base for exterior paints or finishes. These plastering methods are critical for long-lasting building envelopes, reducing maintenance, and preventing dampness-related deterioration. Advanced systems such as polymer-modified, crystalline, fibre-reinforced, and bituminous waterproof plasters are employed based on project budget, climate conditions, and performance expectations in villas, apartments, and commercial buildings.

Consumption Chart Per 1m²

| Plaster Type | Mix Ratio | Thickness | Cement | Sand | Additive / Chemical | Water | Mason | Helper |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cement Polymer Plaster | 1:4 | 15 mm | 0.28 bag (14 kg) | 0.045 m³ | SBR Polymer ~150 ml | 5 – 6 L | 0.24 – 0.27 MD | 0.16 – 0.18 MD |
| Integral Waterproof Plaster | 1:5 | 18 mm | 0.30 bag (15 kg) | 0.052 m³ | Integral Compound ~150 g | 6 – 7 L | 0.26 – 0.28 MD | 0.17 – 0.19 MD |
| Crystalline Waterproof Plaster | 1:4 | 15 mm | 0.28 bag (14 kg) | 0.045 m³ | Crystalline Powder ~250 g | 5 – 6 L | 0.24 – 0.27 MD | 0.16 – 0.18 MD |
| Fibre-Reinforced WP Plaster | 1:4 | 20 mm | 0.36 bag (18 kg) | 0.060 m³ | Polymer 150 ml + Fibre Mesh (0.9 m²) | 6 – 7 L | 0.28 – 0.30 MD | 0.18 – 0.20 MD |
| Bitumen Layer over Cement Plaster | 1:3 | 20 mm + coat | 0.42 bag (21 kg) | 0.065 m³ | Bitumen Coating ~1.25 kg | 7 – 8 L | 0.30 – 0.32 MD | 0.18 – 0.20 MD |
| Polymer Mortar Screed Plaster | 1:3 | 20 mm | 0.42 bag (21 kg) | 0.065 m³ | SBR Polymer ~250 ml | 7 – 8 L | 0.30 – 0.32 MD | 0.18 – 0.20 MD |
| Waterproof Putty Finish (Top Coat) | Ready Mix | 3 mm (only) | — | — | Acrylic Putty ~1.2–1.5 kg | 0.3 L | 0.13 – 0.15 MD | 0.08 – 0.10 MD |

# Floor Screeding Work

| Unit of Measurement | m² |
| --- | --- |
| Mortar Mix | 1:4 Cement Mortar |
| Cement | 0.35–0.50 bags |
| Sand | 0.03–0.04 m³ |
| Thickness | 40–75 mm |
| Labour | 0.20–0.25 Man-day |
| Applicable Standards | IS 2571, IS 1200 |
| Tolerance | ±3 mm |

Floor Screeding Work with 6 mm Jelly – Material & Manpower Consumption (Per 1 m²)

Description:
Floor screeding with 6 mm jelly using 1:3, 1:4, and 1:5 cement-sand mixes at 30 mm, 40 mm, and 50 mm thickness respectively is deployed for internal floor leveling and surface preparation prior to final finishes (tiles, epoxy, vinyl). The jelly addition enhances compaction, reduces shrinkage cracks, and improves surface strength. This structured data enables accurate resource planning, material procurement, cost estimation, and systematic site execution.

Consumption Chart Per 1m²

| Component | 1:3 Mix (30 mm Thick) | 1:4 Mix (40 mm Thick) | 1:5 Mix (50 mm Thick) |
| --- | --- | --- | --- |
| Cement (OPC/PPC) | 0.42 Bag (21 kg) | 0.44 Bag (22 kg) | 0.42 Bag (21 kg) |
| Sand (River Sand) | 0.052 m³ | 0.080 m³ | 0.088 m³ |
| 6 mm Jelly (Chips) | 0.0075 m³ | 0.008 m³ | 0.012 m³ |
| Water | 6 – 8 L | 7 – 9 L | 9 – 11 L |
| Mason | 0.22 – 0.24 Man-day | 0.24 – 0.26 Man-day | 0.25 – 0.28 Man-day |
| Helper | 0.15 – 0.17 Man-day | 0.16 – 0.18 Man-day | 0.18 – 0.20 Man-day |
| Mixer Machine Usage | 0.12 Hr | 0.15 Hr | 0.18 Hr |
| Vibrating Screed (Opt.) | 0.08 Hr | 0.10 Hr | 0.12 Hr |

# Floor Level Bed Work

| Unit of Measurement | m² |
| --- | --- |
| Concrete Grade | M5,M10,M15 |
| Cement | 0.6–0.8 bags ( For M10 ) |
| Sand | 0.04 m³ ( For M10 ) |
| Aggregate | 0.06 m³ ( For M10 ) |
| Thickness | 75–100 mm |
| Applicable Standards | IS 456, IS 1200 |
| Level Tolerance | ±5 mm |

Floor Level Bed Work (PCC) – 100 mm Thick – Material & Manpower Consumption (Per 1 m²)

Description:
Plain Cement Concrete (PCC) floor level beds are executed as sub-base preparation for floors, driveways, and ground floors to provide a uniform, stable, and level surface before finishing layers (screeds, tiles, or industrial coatings). Using M5, M10, and M15 grades allows selection based on structural requirements, load-bearing capacity, and project specifications. This table aids precise material estimation, procurement planning, and labor allocation for efficient site execution and cost management.

Consumption Chart Per 1m²

| Component | M5 (1:5:10) | M10 (1:4:8) | M15 (1:2:4) |
| --- | --- | --- | --- |
| Cement (OPC/PPC) | 0.22 Bag (11 kg) | 0.29 Bag (14.5 kg) | 0.44 Bag (22 kg) |
| Sand (Zone II) | 0.036 m³ | 0.040 m³ | 0.044 m³ |
| Jelly (20 mm) | 0.072 m³ | 0.080 m³ | 0.088 m³ (20 + 10 mm) |
| Water | 15 – 18 L | 18 – 20 L | 20 – 22 L |
| Mason (Level/Finish) | 0.25 – 0.28 MD | 0.26 – 0.30 MD | 0.30 – 0.32 MD |
| Helper | 0.18 – 0.20 MD | 0.20 – 0.22 MD | 0.22 – 0.24 MD |
| Mixer Machine (5–7 CFT) | 0.20 Hr | 0.20 Hr | 0.22 Hr |
| Vibrator | Not Required | Optional (0.08 Hr) | Recommended (0.10 Hr) |

Mass Concrete Work (PCC) – 100 mm Thick – Material & Manpower Consumption (Per 1 m²)

Description:
Mass concrete work using Plain Cement Concrete (PCC) at 100 mm thickness is executed for foundations, base pads, mass flooring, road shoulders, and levelling layers where uniform strength distribution and load transfer are essential. The table below enables accurate material planning, batching, and systematic site execution for M5, M7.5, and M10 mixes, supporting operational efficiency and budget adherence during mass concreting activities.

Consumption Chart Per 1m²

| Component | M5 (1:5:10) | M7.5 (1:4:8) | M10 (1:3:6) |
| --- | --- | --- | --- |
| Cement (OPC / PPC) | 0.22 Bag (11 kg) | 0.27 Bag (13.5 kg) | 0.33 Bag (16.5 kg) |
| Sand (Zone II) | 0.030 m³ | 0.032 m³ | 0.035 m³ |
| Jelly (20 mm) | 0.075 m³ | 0.078 m³ | 0.080 m³ |
| Water | 14 – 16 L | 16 – 18 L | 18 – 20 L |
| Mason | 0.20 – 0.22 Man-day | 0.22 – 0.24 Man-day | 0.24 – 0.26 Man-day |
| Helper | 0.15 – 0.18 Man-day | 0.16 – 0.18 Man-day | 0.18 – 0.20 Man-day |
| Mixing Machine (5–7 CFT) | 0.18 Hr | 0.20 Hr | 0.20 Hr |
| Vibrator (if used) | Not Required | Optional – 0.05 Hr | Optional – 0.06 Hr |

# Vacuum Dewatered Flooring- Site Mix

| Unit of Measurement | m² |
| --- | --- |
| Concrete Grade | M25 |
| Thickness | 100–150 mm |
| Cement Content | 35–45 kg |
| Equipment | VDF Pump, Power Trowel |
| Labour | 0.12–0.18 Man-day |
| Applicable Standards | IS 2571 |

(Using M20 Concrete Mix, OPC/PPC cement, with standard compaction and curing practices)

This matrix outlines standard material and resource consumption norms for VDF flooring at 75 mm, 50 mm, and 25 mm thickness, enabling accurate estimation, planning, and productivity tracking aligned with project delivery excellence.

Consumption Chart Per 1m²

| Component | VDF 75 mm (M20) | VDF 50 mm (M20) | VDF 25 mm (M20) |
| --- | --- | --- | --- |
| Cement (OPC/PPC) | 0.85 Bag (42.5 kg) | 0.57 Bag (28.5 kg) | 0.28 Bag (14 kg) |
| Sand (River Sand) | 0.080 m³ | 0.054 m³ | 0.027 m³ |
| Aggregates (12/20 mm) | 0.100 m³ | 0.067 m³ | 0.034 m³ |
| Water | 18 – 22 L | 12 – 15 L | 6 – 8 L |
| Mason | 0.30 – 0.35 Man-day | 0.22 – 0.25 Man-day | 0.12 – 0.15 Man-day |
| Helper | 0.20 – 0.22 Man-day | 0.15 – 0.18 Man-day | 0.08 – 0.10 Man-day |
| Concrete Mixer Usage | 0.20 Hr | 0.15 Hr | 0.08 Hr |
| Vibrating Screed | 0.15 Hr | 0.10 Hr | 0.05 Hr |
| Vacuum Dewatering Pump | 0.20 Hr | 0.15 Hr | 0.08 Hr |
| Trowelling Machine | 0.25 Hr | 0.18 Hr | 0.10 Hr |

# Vacuum Dewatered Flooring – Ready Mix

| Unit of Measurement | m² |
| --- | --- |
| Concrete Volume | 0.10–0.15 m³ |
| Cement | 38–44 kg |
| Labour | 0.15–0.20 Man-day |
| Surface Finish | Power trowel finish |
| Applicable Standards | IS 456, IS 2571 |
| QC | Surface flatness F-number |

(VDF) using Ready-Mix Concrete (RMC) – Resource Matrix

(For M20 Grade Concrete, using OPC/PPC cement, river sand, and well-graded aggregates, placed by pump or chute as site permits)

This structured matrix enables precise estimation, planning, and on-ground monitoring for VDF using RMC, supporting time-efficient, quality-controlled flooring execution.

Consumption Chart Per 1m²

| Component | VDF 75 mm (M20 RMC) | VDF 50 mm (M20 RMC) | VDF 25 mm (M20 RMC) |
| --- | --- | --- | --- |
| RMC Quantity | 0.075 m³ | 0.050 m³ | 0.025 m³ |
| Water (Additional, if req.) | 1–2 L (adjust on-site) | 0.5–1 L | 0.25–0.5 L |
| Mason | 0.18 – 0.20 Man-day | 0.12 – 0.15 Man-day | 0.07 – 0.09 Man-day |
| Helper | 0.12 – 0.14 Man-day | 0.08 – 0.10 Man-day | 0.04 – 0.06 Man-day |
| Concrete Pump/Transit Mixer | 0.12 Hr | 0.08 Hr | 0.04 Hr |
| Vibrating Screed | 0.15 Hr | 0.10 Hr | 0.05 Hr |
| Vacuum Dewatering Pump | 0.20 Hr | 0.15 Hr | 0.08 Hr |
| Trowelling Machine | 0.25 Hr | 0.18 Hr | 0.10 Hr |

# Cut Lintel Work

| Unit of Measurement | m³ |
| --- | --- |
| Concrete Grade | M15,M20,M25 |
| Steel Reinforcement | 80–110 kg/m³ |
| Cement | 10.5 bags |
| Bearing Length | Minimum 150 mm |
| Applicable Standards | IS 456 |

Description:
Cut lintels provide structural support across door and window openings, ensuring uniform load transfer to masonry walls. This chart provides material consumption and manpower planning for M15 , M20 , M25 Grade concrete in standard lintel sizes (8”×8”, 8”×12”, 9”×9”, 9”×12”), facilitating accurate resource allocation during site execution.

Consumption Chart Per Running Meter

Cut Lintel – Material & Manpower Consumption Per Running Meter (M1) (M15 Grade)

| Component | 8”×8” (200×200 mm) | 8”×12” (200×300 mm) | 9”×9” (225×225 mm) | 9”×12” (225×300 mm) |
| --- | --- | --- | --- | --- |
| Cement (OPC 53 / PPC) | 0.18 Bag (9 kg) | 0.28 Bag (14 kg) | 0.23 Bag (11.5 kg) | 0.32 Bag (16 kg) |
| Sand (Zone II) | 0.018 m³ | 0.026 m³ | 0.022 m³ | 0.030 m³ |
| Jelly (20 mm & 10 mm) | 0.032 m³ | 0.048 m³ | 0.038 m³ | 0.052 m³ |
| Water | 7 – 8 L | 11 – 12 L | 9 – 10 L | 12 – 13 L |
| Mason (Shuttering + Pouring) | 0.15 – 0.17 MD | 0.20 – 0.22 MD | 0.18 – 0.20 MD | 0.22 – 0.24 MD |
| Helper | 0.12 – 0.14 MD | 0.14 – 0.16 MD | 0.13 – 0.15 MD | 0.15 – 0.17 MD |
| Bar Bender | 0.09 – 0.10 MD | 0.11 – 0.12 MD | 0.11 – 0.12 MD | 0.13 – 0.14 MD |
| Shuttering Carpenter | 0.11 – 0.12 MD | 0.14 – 0.15 MD | 0.13 – 0.14 MD | 0.15 – 0.16 MD |
| Mixing Machine | 0.06 Hr | 0.08 Hr | 0.07 Hr | 0.09 Hr |
| Vibrator | 0.04 – 0.05 Hr | 0.06 – 0.07 Hr | 0.05 – 0.06 Hr | 0.07 – 0.08 Hr |

Cut Lintel – Material & Manpower Consumption Per Running Meter (M1) (M20 Grade)

| Component | 8”×8” (200×200 mm) | 8”×12” (200×300 mm) | 9”×9” (225×225 mm) | 9”×12” (225×300 mm) |
| --- | --- | --- | --- | --- |
| Cement (OPC 53) | 0.21 Bag (10.5 kg) | 0.32 Bag (16 kg) | 0.27 Bag (13.5 kg) | 0.36 Bag (18 kg) |
| Sand (Zone II) | 0.016 m³ | 0.024 m³ | 0.020 m³ | 0.027 m³ |
| Jelly (20 mm & 10 mm) | 0.030 m³ | 0.045 m³ | 0.036 m³ | 0.050 m³ |
| Water | 8 – 9 L | 12 – 13 L | 10 – 11 L | 13 – 14 L |
| Mason (Shuttering + Pouring) | 0.15 – 0.18 MD | 0.20 – 0.22 MD | 0.18 – 0.20 MD | 0.22 – 0.24 MD |
| Helper | 0.12 – 0.14 MD | 0.14 – 0.16 MD | 0.13 – 0.15 MD | 0.15 – 0.17 MD |
| Bar Bender | 0.10 MD | 0.12 MD | 0.12 MD | 0.14 MD |
| Shuttering Carpenter | 0.12 MD | 0.15 MD | 0.14 MD | 0.16 MD |
| Mixing Machine | 0.06 Hr | 0.08 Hr | 0.07 Hr | 0.09 Hr |
| Vibrator | 0.05 Hr | 0.07 Hr | 0.06 Hr | 0.08 Hr |

Cut Lintel – Material & Manpower Consumption Per Running Meter (M1) (M25 Grade)

| Component | 8”×8” (200×200 mm) | 8”×12” (200×300 mm) | 9”×9” (225×225 mm) | 9”×12” (225×300 mm) |
| --- | --- | --- | --- | --- |
| Cement (OPC 53) | 0.26 Bag (13 kg) | 0.40 Bag (20 kg) | 0.32 Bag (16 kg) | 0.46 Bag (23 kg) |
| Sand (Zone II) | 0.014 m³ | 0.022 m³ | 0.018 m³ | 0.026 m³ |
| Jelly (20 mm & 10 mm) | 0.027 m³ | 0.041 m³ | 0.034 m³ | 0.046 m³ |
| Water | 8 – 9 L | 12 – 13 L | 10 – 11 L | 13 – 14 L |
| Mason (Shuttering + Pouring) | 0.16 – 0.18 MD | 0.21 – 0.23 MD | 0.19 – 0.21 MD | 0.23 – 0.25 MD |
| Helper | 0.13 – 0.14 MD | 0.15 – 0.17 MD | 0.14 – 0.16 MD | 0.16 – 0.18 MD |
| Bar Bender | 0.10 – 0.11 MD | 0.12 – 0.13 MD | 0.11 – 0.12 MD | 0.13 – 0.14 MD |
| Shuttering Carpenter | 0.12 – 0.13 MD | 0.14 – 0.15 MD | 0.13 – 0.14 MD | 0.15 – 0.16 MD |
| Mixing Machine | 0.07 Hr | 0.09 Hr | 0.08 Hr | 0.10 Hr |
| Vibrator | 0.05 – 0.06 Hr | 0.07 – 0.08 Hr | 0.06 – 0.07 Hr | 0.08 – 0.09 Hr |

# Tilling Work - Using Adhesive

| Unit of Measurement | m² |
| --- | --- |
| Adhesive | 4–5 kg/m² |
| Grout | 0.20 kg/m² |
| Labour | 0.20–0.25 Man-day |
| Joint Width | 2–3 mm |
| Applicable Standards | IS 15477 |

Wall Tiling (Dado) Work – Material & Manpower Consumption per 1 m²

Description:
Wall tiling (dado work) using tile adhesive requires systematic planning of material, adhesive quantities, water for mixing, and labor allocation to ensure consistent alignment, levelling, and finish quality across small, medium, and large-format tiles. Accurate estimation is crucial for achieving uniform joints, high productivity rates, and reducing wastage on site, enabling fast-track project delivery in residential, commercial, and hospitality segments.

The following table provides practical consumption benchmarks per 1 m², ensuring alignment with advanced tiling methodologies for your ongoing and upcoming projects:

Consumption Chart Per 1 m² Using Adhesive

| Component | Small Tile (300–500 mm) | Medium Tile (600–900 mm) | Large Tile (1000 mm & above) |
| --- | --- | --- | --- |
| Tile Requirement | 1 m² + 5% wastage | 1 m² + 5% wastage | 1 m² + 5% wastage |
| Tile Adhesive | 4.5 – 5.0 kg | 5.0 – 6.0 kg | 6.5 – 8.0 kg |
| Water (for adhesive) | 1.2 – 1.4 L | 1.4 – 1.6 L | 1.6 – 1.8 L |
| Mason (Tiling) | 0.22 – 0.25 MD | 0.25 – 0.28 MD | 0.28 – 0.32 MD |
| Helper | 0.15 – 0.18 MD | 0.18 – 0.20 MD | 0.20 – 0.22 MD |
| Grout (if applicable) | 80 – 100 g | 70 – 90 g | 50 – 70 g |
| Levelling Tools | Not mandatory | Optional spacers | Required: clips & tile spacers |
| Surface Preparation | Plumbed cement wall | Smooth levelled surface | RCC/plaster + primer/bonding agent |

Floor Tiling – Adhesive Fixing (Per 1 m²)

Description:
Adhesive-based floor tiling is a precision-driven method used for fast and high-finish tile installations. It enhances adhesion, reduces setting time, and improves overall alignment compared to traditional cement-sand bedding. Consumption rates vary depending on tile size, surface preparation, and levelling tools used. This method ensures minimal wastage, enhanced bonding, and better joint uniformity across small, medium, and large-format tiles.

The table below details the material and manpower requirements per square meter, optimized for execution efficiency and resource allocation across diverse site conditions.

Consumption Chart Per 1 m² Using Adhesive

| Component | Small Tile(<300–500 mm) | Medium Tile(600–900 mm) | Large Tile(1000 mm & above) |
| --- | --- | --- | --- |
| Tile Requirement | 1 m² + 5% wastage | 1 m² + 5% wastage | 1 m² + 5% wastage |
| Tile Adhesive | 4.0 – 4.5 kg | 4.5 – 5.5 kg | 6.0 – 7.5 kg |
| Water (for adhesive) | 1.0 – 1.2 L | 1.2 – 1.4 L | 1.4 – 1.6 L |
| Mason (Tiling) | 0.20 – 0.22 MD | 0.22 – 0.25 MD | 0.25 – 0.30 MD |
| Helper | 0.12 – 0.15 MD | 0.15 – 0.18 MD | 0.18 – 0.20 MD |
| Grout | 70 – 90 gm | 60 – 80 gm | 50 – 70 gm |
| Levelling Tools | Optional spacers | Recommended: spacers & laser level | Mandatory: clips, spacers, vibration pad |
| Surface Preparation | Screed or IPS finished surface | Smooth cement bed or screed | Flat RCC / Self-levelled screed |

# Tiling Work- Using Cement Mortar

Wall Dadoing – Using Cement Mortar (Wet Fixing) – Per 1 m²

| Unit of Measurement | m² |
| --- | --- |
| Mortar Mix | 1:4 CM |
| Cement | 0.30 bags |
| Sand | 0.02 m³ |
| Labour | 0.25–0.30 Man-day |
| Applicable Standards | IS 2212 |

Description:
Wet fixing for wall dadoing with cement mortar is a robust method suitable for washrooms, kitchens, utility areas, and external skirting where durability, impact resistance, and moisture tolerance are critical. This method allows alignment adjustments during installation while ensuring a strong bond with plastered walls. Material and labor consumption vary by tile size, mortar thickness, and finish requirements, impacting productivity and project schedules.

The following table outlines material consumption and manpower requirements per square meter under practical site conditions:

Consumption Chart Per 1 m² Using Cement Mortar

| Component | Small Tile(<300–600 mm) | Medium Tile(600–800 mm) | Large Tile(800 mm & above) |
| --- | --- | --- | --- |
| Tile Requirement | 1 m² + 5% wastage | 1 m² + 5% wastage | 1 m² + 5% wastage |
| Mortar Thickness | 15 – 20 mm | 20 – 25 mm | 25 – 30 mm |
| Cement (OPC/PPC) | 0.18 – 0.22 Bag (9–11 kg) | 0.22 – 0.26 Bag (11–13 kg) | 0.26 – 0.30 Bag (13–15 kg) |
| Sand (River Sand) | 0.025 – 0.030 m³ | 0.030 – 0.035 m³ | 0.035 – 0.040 m³ |
| Water (Mixing & Wiping) | 4 – 5 L | 5 – 6 L | 6 – 7 L |
| Mix Ratio (Cement:Sand) | 1:4 or 1:5 | 1:4 or 1:5 | 1:4 or 1:5 |
| Mason (Tiling) | 0.22 – 0.25 MD | 0.25 – 0.28 MD | 0.28 – 0.32 MD |
| Helper | 0.15 – 0.18 MD | 0.18 – 0.20 MD | 0.20 – 0.22 MD |
| Grout (Cement Slurry) | 60 – 80 gm | 70 – 90 gm | 80 – 100 gm |
| Tools Required | Trowel, Level Board, Spacers | Tile Spacers, Rubber Mallet | Clips, Spacers, Alignment Systems |

Floor Tiling – Using Cement Mortar (Wet Fixing) – Per 1 m²

Description:
Wet fixing with cement mortar is a conventional, durable floor tiling method suited for a wide range of projects, including residential, commercial, and industrial floors requiring robust bonding with substrates. It involves setting tiles on a fresh mortar bed, ensuring levels and slope control during installation. The consumption of materials and manpower varies with tile size and mortar bed thickness, requiring careful planning to optimize labor productivity and material utilization.

The following table outlines material consumption and manpower requirements per square meter for different tile sizes under typical site conditions:

Consumption Chart Per 1 m² Using Cement Mortar

| Component | Small Tile(<300–600 mm) | Medium Tile(600–800 mm) | Large Tile(800 mm & above) |
| --- | --- | --- | --- |
| Tile Requirement | 1 m² + 5% wastage | 1 m² + 5% wastage | 1 m² + 5% wastage |
| Mortar Thickness | 25 – 30 mm | 30 – 40 mm | 40 – 50 mm |
| Cement (OPC/PPC) | 0.22 – 0.26 Bag (11–13 kg) | 0.25 – 0.30 Bag (13–15 kg) | 0.30 – 0.35 Bag (15–17.5 kg) |
| Sand (River Sand) | 0.035 – 0.045 m³ | 0.045 – 0.055 m³ | 0.055 – 0.065 m³ |
| Water (Mixing & Wiping) | 6 – 7 L | 7 – 8 L | 8 – 9 L |
| Mix Ratio (Cement:Sand) | 1:4 or 1:5 | 1:4 or 1:5 | 1:4 or 1:5 |
| Mason (Tiling) | 0.25 – 0.28 MD | 0.28 – 0.32 MD | 0.32 – 0.36 MD |
| Helper | 0.18 – 0.22 MD | 0.22 – 0.25 MD | 0.25 – 0.28 MD |
| Grout (Cement Slurry) | 80 – 100 gm | 70 – 90 gm | 60 – 80 gm |
| Tools Required | Trowel, Rubber Mallet, Level Board | Tile Spacers, Straight Edge | Clips, Spacers, Tile Lifter (for heavy tiles) |

# Granite Flooring Work

Granite Flooring Work – Using Cement Mortar – Per 1 m²

| Unit of Measurement | m² |
| --- | --- |
| Thickness | 18–20 mm |
| Mortar Mix | 1:4 CM |
| Cement | 0.35 bags |
| Labour | 0.25–0.35 Man-day |
| Applicable Standards | IS 14223 |

Description:
Granite flooring installation using cement mortar is a premium flooring solution for high-traffic, luxury, and public spaces, providing durability, high load-bearing capacity, and a polished aesthetic. Execution involves precise mortar preparation, bed leveling, and proper alignment of granite slabs to ensure seamless joints and long-term performance. Consumption and manpower vary based on slab size, thickness, and jointing requirements, affecting material planning, curing schedules, and project timelines.

The table below provides practical site-level consumption norms and manpower requirements for effective planning and monitoring:

Consumption Chart Per 1 m²

| Component | Small Slab(cut-to-size tiles) | Medium Slab(standard slabs) | Large Slab(jumbo slabs) |
| --- | --- | --- | --- |
| Granite Requirement | 1 m² + 5% wastage | 1 m² + 8% wastage | 1 m² + 10% wastage |
| Granite Thickness | 18 – 20 mm | 18 – 20 mm | 20 – 25 mm |
| Mortar Thickness | 25 – 30 mm | 30 – 50 mm | 40 – 60 mm (adjustable) |
| Cement (OPC/PPC) | 0.30 – 0.34 Bag (15 – 17 kg) | 0.32 – 0.36 Bag (16 – 18 kg) | 0.36 – 0.40 Bag (18 – 20 kg) |
| Sand (River Sand) | 0.045 – 0.055 m³ | 0.055 – 0.065 m³ | 0.065 – 0.075 m³ |
| Water (Mix + Curing) | 7 – 8 L | 8 – 10 L | 10 – 12 L |
| Cement Slurry / Paste | 1.0 – 1.2 kg | 1.2 – 1.5 kg | 1.5 – 2.0 kg |
| Mason (Fixing) | 0.38 – 0.42 Man-day | 0.35 – 0.40 Man-day | 0.40 – 0.45 Man-day |
| Helper | 0.22 – 0.25 Man-day | 0.22 – 0.25 Man-day | 0.25 – 0.30 Man-day |
| Grouting (Cement) | 100 – 120 gm | 80 – 100 gm | 60 – 80 gm |
| Tools Required | Trowel, Level Bar, Rubber Mallet | Cutter, Spacer, Edge Tools | Tile Lifters, Vacuum Cups, Edge Polisher |

Granite Wall Cladding Using Adhesive Fixing – Per 1 m²

Description:
Granite wall cladding with adhesive fixing offers a clean, efficient, and structurally lighter alternative to cement mortar methods, ensuring a strong bond, uniform finish, and reduced installation time. This method is ideal for interior and exterior feature walls, lift lobbies, staircases, and façade cladding, minimizing dead load, reducing wastage, and delivering premium aesthetics with fewer hollow patches. Adhesive fixing also allows precise joint alignment, reduces water curing requirements, and supports a faster project cycle for luxury and high-rise projects.

The following table details material and manpower consumption per 1 m² for effective BOQ, procurement, and execution alignment:

Consumption Chart Per 1 m²

| Component | Small Slab (cut-to-size tiles) | Medium Slab (standard slabs) | Large Slab (jumbo slabs) |
| --- | --- | --- | --- |
| Granite Requirement | 1 m² + 5% wastage | 1 m² + 8% wastage | 1 m² + 10% wastage |
| Granite Thickness | 15 – 18 mm | 18 – 20 mm | 20 – 25 mm |
| Adhesive Consumption | 4.0 – 5.0 kg | 5.0 – 6.5 kg | 6.5 – 8.0 kg |
| Water (for adhesive) | 0.8 – 1.0 L | 1.0 – 1.2 L | 1.2 – 1.4 L |
| Primer/Slurry Coat | Optional, ~0.4 kg | Optional, ~0.5 kg | Optional, ~0.7 kg |
| Mason (Fixing) | 0.28 – 0.32 Man-day | 0.30 – 0.35 Man-day | 0.35 – 0.40 Man-day |
| Helper | 0.18 – 0.20 Man-day | 0.20 – 0.22 Man-day | 0.22 – 0.25 Man-day |
| Grouting (epoxy/cement) | 60 – 80 gm | 60 – 80 gm | 50 – 70 gm |
| Tools Required | Notched Trowel, Rubber Mallet, Spacer | Clips, Spacers, Spirit Level | Vacuum Lifter, Alignment System, Spacers |

# Sunken Filling

Toilet Sunken and Mass Earth Filling – Per 1 m³

| Unit of Measurement | m³ |
| --- | --- |
| Material | Brick Bats / Foam Concrete |
| Cement | 4–5 bags |
| Labour | 0.40–0.60 Man-day |
| Applicable Standards | IS 3495 |
| QC Requirement | Waterproofing layer mandatory below filling |

Description:
Sunken filling in toilets is a critical activity in high-quality construction to ensure proper slope management for drainage lines while providing robust bedding support. Mass earth filling, on the other hand, is employed to raise plinth levels and prepare stable platforms for further construction, ensuring compaction for long-term stability. The execution of these activities impacts subsequent waterproofing, tiling, and plumbing precision, requiring careful coordination of manpower, equipment, and material logistics.

The chart below outlines practical material and manpower benchmarks per 1 m³, enabling precise BOQ preparation, vendor billing, and activity scheduling under your Total Sheet and project management workflows.

Consumption Chart – Per 1 m³

| Component | Toilet Sunken Filling | Mass Earth Filling |
| --- | --- | --- |
| Labor (Man-days) | 0.30 – 0.40 | 0.20 – 0.25 |
| Material Type | Broken Brick Jelly / Stone Chips + Sand | Selected Earth / Murum |
| Brick Jelly / Rubble | 0.70 – 0.85 m³ (base) | Not Applicable |
| Sand / Soil (Blinding) | 0.15 – 0.20 m³ (top levelling) | Full volume with selected earth |
| Excavator / Loader | 0.08 – 0.10 Hr (optional) | 0.08 – 0.10 Hr |
| Truck / Tipper (6–10 m³) | 0.10 – 0.12 Trips | 0.10 – 0.12 Trips |
| Compaction Equipment | Hand Rammer / Plate Compactor (0.10 – 0.12 Hr) | Vibratory Roller / Plate (0.10 Hr) |
| Water for Compaction | 10 – 15 L (sprinkling) | 20 – 25 L (layered compaction) |
| Layer Thickness | Max 150 mm / layer | 200 mm / layer |
| Slope Treatment | Required for drainage pipe slope | Not Required |
| Pipe Protection | Sand or lean mortar bedding | Not Applicable |
| Waterproofing | Done post-filling (separate item) | Not Applicable |

# Painting Work

| Unit of Measurement | m²,Sqft |
| --- | --- |
| Labour | 0.15–0.20 Man-day |
| Applicable Standards | IS 2395 |

Internal Ceiling Painting – Per 1 m² (Standard 3-Coat System)

Description:
Internal ceiling painting enhances light reflectivity, ambiance, and durability of spaces while maintaining a dust-free, stain-resistant surface. A standard 3-coat system involves surface preparation with putty, priming for sealing and adhesion, followed by two top coats to ensure finish consistency and longevity. Proper manpower planning, accurate material consumption estimation, and adherence to curing/drying times directly affect project timelines, rework rates, and final aesthetics in residential, commercial, and hospitality projects.

The following consumption chart standardizes your paint type-wise material and manpower planning across varied finish grades.

Consumption Chart – Per  1 m²

| Type of Paint System | Putty (White Cement) | Primer (Water-based) | Top Coat (2 Coats) | Mason/Painter | Helper | Water (for dilution) |
| --- | --- | --- | --- | --- | --- | --- |
| White Cement Paint | 0.30 – 0.35 kg | 0.10 – 0.12 L | 0.15 – 0.18 kg | 0.15 – 0.18 MD | 0.10 – 0.12 MD | 0.30 – 0.35 L |
| Distemper (Dry/Soft Finish) | 0.35 – 0.40 kg | 0.12 – 0.14 L | 0.18 – 0.22 kg | 0.18 – 0.20 MD | 0.12 – 0.14 MD | 0.40 – 0.45 L |
| Plastic Emulsion Paint (Standard) | 0.40 – 0.45 kg | 0.10 – 0.12 L | 0.20 – 0.25 L | 0.20 – 0.22 MD | 0.12 – 0.14 MD | 0.35 – 0.40 L |
| Premium Emulsion (Velvet/Soft Sheen) | 0.45 – 0.50 kg | 0.10 – 0.12 L | 0.22 – 0.28 L | 0.22 – 0.25 MD | 0.14 – 0.16 MD | 0.40 – 0.45 L |
| Royal / Luxury Emulsion | 0.50 – 0.55 kg | 0.10 – 0.12 L | 0.25 – 0.30 L | 0.25 – 0.28 MD | 0.16 – 0.18 MD | 0.45 – 0.50 L |

Internal Wall Painting – Per 1 m² (3-Coat System)

Description:
Internal wall painting enhances interior quality, light reflection, and aesthetic value while protecting surfaces from dust and moisture penetration. A standard 3-coat system includes putty application for levelling, a primer for sealing and adhesion, and two top coats for a consistent, durable finish. Selecting the appropriate paint system based on project type (residential, commercial, luxury) impacts project timelines, client satisfaction, and maintenance cycles.

Proper estimation and execution ensure cost control, reduced rework, and improved productivity in your sites and billing control systems.

Consumption Chart – Per  1 m²

| Paint System | Putty (White Cement) | Primer (Water-based) | Top Coat (2 Coats) | Painter | Helper | Water (Dilution) |
| --- | --- | --- | --- | --- | --- | --- |
| White Cement Paint | 0.30 – 0.35 kg | 0.10 – 0.12 L | 0.15 – 0.18 kg | 0.18 – 0.20 MD | 0.12 – 0.14 MD | 0.30 – 0.35 L |
| Dry Distemper | 0.35 – 0.40 kg | 0.12 – 0.14 L | 0.18 – 0.22 kg | 0.20 – 0.22 MD | 0.14 – 0.16 MD | 0.40 – 0.45 L |
| Plastic Emulsion (Standard) | 0.40 – 0.45 kg | 0.10 – 0.12 L | 0.20 – 0.25 L (per coat) | 0.22 – 0.25 MD | 0.14 – 0.16 MD | 0.40 – 0.50 L |
| Premium Emulsion | 0.45 – 0.50 kg | 0.10 – 0.12 L | 0.22 – 0.28 L (per coat) | 0.25 – 0.28 MD | 0.16 – 0.18 MD | 0.45 – 0.55 L |
| Luxury / Royal Emulsion | 0.50 – 0.55 kg | 0.10 – 0.12 L | 0.25 – 0.30 L (per coat) | 0.28 – 0.30 MD | 0.18 – 0.20 MD | 0.50 – 0.60 L |

External Ceiling Painting – Per 1 m² (3-Coat System)

Description:
External ceiling painting enhances durability and aesthetics while providing protection against rain, UV exposure, fungus growth, and pollution. A 3-coat system—comprising putty, primer, and two top coats—ensures extended lifecycle, reduced maintenance, and consistent surface protection, especially critical in coastal or high-rainfall zones.

Selection of paint system should align with client budgets, climatic conditions, and project positioning (premium vs standard), ensuring efficiency in application cycles and labor optimization.

Consumption Chart – Per  1 m²

| Paint System | Putty (Acrylic/Cement) | Primer (Weather-resistant) | Top Coat (2 Coats) | Painter | Helper | Water (Dilution) |
| --- | --- | --- | --- | --- | --- | --- |
| Exterior Cement Paint | 0.30 – 0.35 kg | 0.10 – 0.12 L | 0.18 – 0.22 kg | 0.20 – 0.22 MD | 0.12 – 0.14 MD | 0.30 – 0.35 L |
| Acrylic Exterior Emulsion | 0.35 – 0.40 kg | 0.10 – 0.12 L | 0.20 – 0.25 L (per coat) | 0.22 – 0.25 MD | 0.14 – 0.16 MD | 0.35 – 0.40 L |
| Weatherproof Emulsion | 0.40 – 0.45 kg | 0.10 – 0.12 L | 0.22 – 0.28 L (per coat) | 0.25 – 0.28 MD | 0.16 – 0.18 MD | 0.40 – 0.50 L |
| Elastomeric / Anti-fungal Coating | 0.45 – 0.50 kg (optional) | 0.10 – 0.12 L | 0.25 – 0.30 L (per coat) | 0.28 – 0.30 MD | 0.18 – 0.20 MD | 0.45 – 0.55 L |

External Wall Painting – Per 1 m² (Standard 3-Coat System)

Description:
External wall painting safeguards the building envelope against rain, UV radiation, dust, and carbonation while enhancing its aesthetic appeal. A 3-coat system—putty, primer, and two top coats—ensures extended protection, uniform finish, and reduced maintenance cycles, essential for coastal or harsh weather zones.

Paint system selection should align with client budget, weather conditions, and maintenance preferences while ensuring effective manpower utilization and optimal material consumption on-site. Surface preparation, curing intervals, and dilution management are critical to achieving a defect-free and durable finish.

Consumption Chart – Per  1 m²

| Paint System | Putty (Acrylic/Cement) | Primer (Exterior Grade) | Top Coat (2 Coats) | Painter | Helper | Water (Dilution) |
| --- | --- | --- | --- | --- | --- | --- |
| Cement-Based Paint | 0.35 – 0.40 kg | 0.10 – 0.12 L | 0.18 – 0.22 kg | 0.20 – 0.22 MD | 0.12 – 0.14 MD | 0.30 – 0.35 L |
| Acrylic Exterior Emulsion | 0.40 – 0.45 kg | 0.10 – 0.12 L | 0.20 – 0.25 L (per coat) | 0.22 – 0.25 MD | 0.14 – 0.16 MD | 0.35 – 0.40 L |
| Weatherproof Emulsion | 0.45 – 0.50 kg | 0.10 – 0.12 L | 0.22 – 0.28 L (per coat) | 0.25 – 0.28 MD | 0.16 – 0.18 MD | 0.40 – 0.50 L |
| Elastomeric Paint | 0.50 – 0.55 kg (if applied) | 0.10 – 0.12 L | 0.25 – 0.30 L (per coat) | 0.28 – 0.30 MD | 0.18 – 0.20 MD | 0.45 – 0.55 L |
| Anti-Carbonation Coating | Optional | 0.12 – 0.14 L | 0.30 – 0.35 L (2 thick coats) | 0.30 – 0.35 MD | 0.20 – 0.22 MD | 0.50 – 0.60 L |

# Anti-Termite Treatment

| Unit of Measurement | m² |
| --- | --- |
| Chemical | Chlorpyrifos Emulsion |
| Consumption | 5 L/m² (Horizontal), 7.5 L/m² (Vertical) |
| Labour | 0.05–0.Man-day |
| Applicable Standards | IS 6313 |

Description:
Anti-termite treatment is a critical preventive measure in both pre- and post-construction phases to safeguard structural integrity against subterranean termite attacks. It involves the application of insecticidal chemicals, either Chlorpyrifos 20% EC or Imidacloprid 30.5 SC, diluted with water and applied using low-pressure spraying, injection, or trenching techniques.

The process must be executed systematically before or after key milestones such as PCC, flooring, wall construction, or installation of woodworks. All applications should comply with IS 6313 (Part II & III) and ensure operator PPE safety, chemical dilution accuracy, and thorough penetration into targeted soil or structures.

Materials, Chemical & Labor Consumption per 1 m²

| Treatment Type | Chemical(Chlorpyrifos 20% EC or Imidacloprid 30.5 SC) | Diluted Chemical(with Water) | Labor(Man-days/m²) | Application Method | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1. Below Foundation Footing (Pre-construction) | 0.075 – 0.085 L | 5.0 – 5.5 L | 0.08 – 0.10 | Low-pressure spraying | Over compacted soil before PCC |
| 2. Wall Trenches (Pre-construction) | 0.075 – 0.085 L | 5.0 – 5.5 L | 0.10 – 0.12 | Spraying both trench sides | Base and vertical trench faces |
| 3. Plinth Filling (Before PCC) | 0.075 L/m² | 5.0 L/m² | 0.08 – 0.10 | Horizontal surface spraying | Even coverage after compaction |
| 4. Junction of Wall & Floor | 0.05 L/m² | 2.5 – 3.0 L/m² | 0.06 – 0.08 | Spraying at wall-floor junction | Establishes vertical chemical barrier |
| 5. Post-Construction – Internal Perimeter | 0.075 – 0.085 L/m² | 5.0 – 5.5 L/m² | 0.12 – 0.15 | Drilling + injection + sealing | 12 mm dia holes at 300 mm c/c intervals |
| 6. External Periphery Treatment | 0.075 – 0.085 L/m² | 5.0 – 5.5 L/m² | 0.10 – 0.12 | Trench + side spraying | 300 mm deep trench outside plinth |
| 7. Wooden Fixtures (Post Installation) | 0.025 – 0.030 L per piece | Direct | 0.02 – 0.04 per point | Drill-inject-plug method | For door frames, windows, wooden panels |

# Granite Stone Masonry for Foundation

| Unit of Measurement | m³ |
| --- | --- |
| Mortar Mix | 1:6 CM |
| Cement | 2.0–2.5 bags |
| Sand | 0.40 m³ |
| Labour | 1.20–1.40 Man-day |
| Applicable Standards | IS 1597 (Part 1) |

Foundation Work – Granite Stone Masonry (Per 1 m³)

Description:
Granite stone masonry for foundations is a durable and load-bearing construction method widely used in residential and infrastructure projects. The masonry is bonded with cement mortar, commonly in 1:6 or 1:8 ratios, depending on structural load and specification. Stones are either rough dressed or hammer-finished, and laid in a stretcher bond with proper interlocking. The preparation, placement, and alignment require skilled masons and quality-controlled mortar.

This table outlines the material and labor consumption per 1 m³ of masonry work, considering dry-to-wet volume conversion and best-practice site conditions.

Consumption Chart (Per 1 m³)

| Component | CM 1:6 Ratio | CM 1:8 Ratio | Remarks |
| --- | --- | --- | --- |
| Granite Stone | 1.10 – 1.15 m³ | 1.10 – 1.15 m³ | Rough dressed / hammer-finished for bedding |
| Cement (OPC/PPC) | 0.25 – 0.28 Bags (12–14 kg) | 0.18 – 0.22 Bags (9–11 kg) | For mortar preparation |
| Sand (River Sand) | 0.35 – 0.38 m³ | 0.35 – 0.38 m³ | Clean and well-graded |
| Water | 25 – 30 Liters | 20 – 25 Liters | For mortar mix consistency |
| Mason | 0.50 – 0.60 Man-day | 0.50 – 0.60 Man-day | For bedding, leveling, and alignment |
| Helper | 0.40 – 0.45 Man-day | 0.40 – 0.45 Man-day | For mixing, lifting, and setting assistance |
| Mortar Mix Volume | ~0.28 m³ (incl. wastage) | ~0.25 m³ (incl. wastage) | Includes 33% dry-to-wet volume expansion |

For Mass Work - Material & Manpower (Per 1 m³)

Description:
Mass stone masonry is used for non-load-bearing walls, compound walls, boundary walls, and retaining structures where aesthetics are secondary to volume filling and cost-effectiveness. Stones used are irregular or roughly dressed, hammer finished at the site, and bonded with cement mortar in 1:6 or 1:8 ratios depending on structural and project cost requirements.

The table below outlines material and manpower consumption per 1 m³, factoring dry-to-wet bulking, site productivity, and standard practices in mass masonry execution.

Consumption Chart (Per 1 m³)

| Component | CM 1:6 Ratio | CM 1:8 Ratio | Remarks |
| --- | --- | --- | --- |
| Granite Stone | 1.15 – 1.20 m³ | 1.15 – 1.20 m³ | Irregular / rough stone; hammer dressed at site |
| Cement (OPC/PPC) | 0.22 – 0.26 Bags (11–13 kg) | 0.16 – 0.20 Bags (8–10 kg) | Lower due to reduced bonding requirement |
| Sand (River Sand) | 0.32 – 0.35 m³ | 0.32 – 0.35 m³ | Medium-fine, well-graded sand |
| Water | 20 – 25 Liters | 18 – 22 Liters | As per mortar workability requirements |
| Mason | 0.40 – 0.50 Man-day | 0.38 – 0.45 Man-day | Based on mass laying productivity |
| Helper | 0.30 – 0.40 Man-day | 0.30 – 0.38 Man-day | Assists in lifting, setting, and mortar preparation |
| Mortar Mix Volume | ~0.25 – 0.28 m³ (incl. wastage) | ~0.22 – 0.25 m³ (incl. wastage) | Includes 33% dry-to-wet volume expansion |

# Road Work Project

# Site Clearance and Stripping of Top Soil

Description:

Removal of vegetation, debris, and topsoil from the entire park area to a depth of approximately 200 mm.

Consumption Chart (per 1 m²):

| Component | Unit | Qty/1m³ | Remarks |
| --- | --- | --- | --- |
| Unskilled Labour | Mandays | 0.25 | Clearing, collection, stacking |
| Skilled Labour | Mandays | 0.05 | Machinery coordination |
| Excavator (Poclain) | Hour | 0.015 | For soil removal |
| Tipper Truck (5 m³) | Trip | 0.05 | Transporting spoil to dump site |
| Diesel (for machines) | Litres | 1.5 | Excavator + tipper |
| Water (dust control) | Litres | 10 | Surface spraying |
| Tools & PPE | LS | Incl. | Shovels, sickles, safety gear |
| Supervision | LS | Incl. | Engineering supervision |

# Excavation of Unsuitable Material

Description:

Excavation of soft/unsuitable material to 300 mm depth, transportation of spoils, compaction of exposed subgrade for NTS readiness.

Consumption Chart (per 1 m³):

| Component | Unit | Qty/1m³ | Remarks |
| --- | --- | --- | --- |
| Excavator | Hour | 0.10 | Earth excavation |
| Tipper Truck (5 m³) | Trip | 0.20 | Haul to dump area |
| Labour (Excavation) | Mandays | 0.15 | Trimming, marking, guiding |
| Labour (Compaction) | Mandays | 0.05 | OMC watering and roller coordination |
| Water (OMC moisture) | Litres | 20 | For base surface preparation |
| Roller Compactor | Hour | 0.05 | Sub-base compaction |
| Diesel | Litres | 2.5 | Total for all machinery |
| Tools & PPE | LS | Incl. | Safety gear |

# NTS Treatment on Exposed Surfaces

Description:

Spray application of Nanotechnology Solution on exposed and compacted areas (approx. 5000 m²/day capacity) for dust and erosion control.

Consumption Chart (per 1 m³):

| Component | Unit | Qty/1m³ | Remarks |
| --- | --- | --- | --- |
| NTS Solution (concentrate) | Litres | 2.0 | Mix ratio per supplier spec |
| Mixing Water | Litres | 10.0 | For dilution & even spray |
| Skilled Labour (Spray Ops) | Mandays | 0.15 | Handling NTS + spraying |
| Unskilled Labour | Mandays | 0.25 | Support staff |
| Sprayer Machine | Hour | 0.01 | Motorized or manual sprayer |
| Diesel (for sprayer/pump) | Litres | 0.5 | Estimated |
| PPE (gloves, masks, etc.) | LS | Incl. | As per safety norms |
| Supervision/Quality Check | LS | Incl. | Daily testing and record-keeping |

# Backfilling with Laterite

Description:

Layered backfilling using laterite in 150 mm lifts, compacted to 100% B.S. density at Optimum Moisture Content (OMC), total thickness = 750 mm.

Consumption Chart (per 1 m³):

| Component | Unit | Qty/1m³ | Remarks |
| --- | --- | --- | --- |
| Laterite Soil (borrowed) | m³ | 1.05 | Allowance for compaction loss |
| Loader | Hour | 0.05 | Soil loading into tippers |
| Tipper Truck (5 m³) | Trip | 0.21 | Material transport |
| Labour (Spreading) | Mandays | 0.15 | Manual leveling |
| Labour (Compaction Assist) | Mandays | 0.05 | Hose/water spraying |
| Water (OMC requirement) | Litres | 20 | For compaction |
| Vibratory Roller | Hour | 0.08 | For layer-wise compaction |
| Diesel | Litres | 2.5 | Combined for loader, roller, trucks |
| PPE and Safety | LS | Incl. | Helmets, boots, reflective vests |
| Testing (FDD, Moisture) | LS | Incl. | Compaction control testing |

# NTS Treatment on Compacted Laterite

Description:

Final surface treatment with NTS on top compacted fill and side slopes to ensure long-term performance, water resistance, and surface bonding.

Consumption Chart (per 1 m³):

| Component | Unit | Qty/1m³ | Remarks |
| --- | --- | --- | --- |
| NTS Solution (concentrate) | Litres | 2.5 | Slightly higher for compacted porous fill |
| Mixing Water | Litres | 15.0 | Dilution and uniform spray |
| Skilled Labour | Mandays | 0.18 | Spraying and quality check |
| Unskilled Labour | Mandays | 0.27 | Hose handling, moving equipment |
| Spray Equipment | Hour | 0.015 | Medium-pressure sprayer |
| Diesel | Litres | 0.5 | Pump/sprayer machine |
| PPE | LS | Incl. | Compliance per site safety |
| Site Engineer (QC) | LS | Incl. | Testing spray uniformity & setting |

# MIVAN (Aluminium) Shuttering

| Unit of Measurement | M2, Sqft |
| --- | --- |
| Material | Aluminium |
| types | Standard, jumping and rail climbing |
| Applicable Standards | IS 14687: 1999 |

Description:
This chart summarizes the system inventory, consumables, manpower, equipment, and time benchmarks for MIVAN aluminium formwork in a typical high-rise monolithic RCC structure (walls + slab).
The cycle is based on 4–6 days/floor, trained crews, and ambient 25–35 °C. All metrics are expressed per m² of shuttered area per casting cycle, excluding Ready-Mix Concrete supply and reinforcement works.

A) Reusable System Inventory (in circulation; not consumed)

| Item | Requirement per m² |
| --- | --- |
| Aluminium wall/slab panels | 1.00 – 1.10 m² |
| Pins & wedges | 8 – 12 sets |
| Wall tie rods | 1.2 – 1.8 rods |
| Tie cones/end caps (reusable pool) | 0.15 – 0.25 sets |
| Primary deck beams (aluminium) | 0.8 – 1.2 lm |
| Secondary deck beams (aluminium) | 1.6 – 2.4 lm |
| Adjustable props (initial propping) | 0.50 – 0.70 props |

B) One-Time Use / Consumables (per casting cycle)

| Item | Consumption per m² |
| --- | --- |
| Release/formwork oil | 0.08 – 0.12 L |
| Grout/mortar for tie-hole reinstatement | 0.08 – 0.12 kg |
| PVC sleeves (loss) | 0.02 – 0.04 m |
| Tie cones/end caps (loss/attrition) | 0.02 – 0.05 sets |
| Pins & wedges (loss/attrition) | 0.05 – 0.12 sets |
| Joint tapes/foam | 0.20 – 0.30 m |
| Curing compound (if used) | 0.10 – 0.15 L |
| Cleaning consumables | 0.02 – 0.04 pcs |

C) Manpower – Full Cycle (Shuttering → Casting → De-shuttering)

| Role | Man-days per m² |
| --- | --- |
| MIVAN carpenters | 0.10 – 0.16 |
| Helpers/Riggers | 0.07 – 0.10 |
| Concreting crew (excl. RMC) | 0.04 – 0.07 |
| QA/QC & survey | 0.01 – 0.02 |
| Safety/housekeeping | 0.01 – 0.02 |
| Total manpower envelope | 0.23 – 0.37 man-days/m² |

D) Equipment / Plant Time (per casting)

| Equipment | Time per m² |
| --- | --- |
| Tower crane/material hoist | 3 – 5 min |
| Needle vibrator (placement) | 0.8 – 1.2 min |
| Power trowel (slab zones) | 0.5 – 0.8 min |
| Spray application (release oil) | 0.3 – 0.5 min |

E) Time & Striking Envelope

| Element | Striking Time |
| --- | --- |
| Walls/columns (sides) | 18 – 24 h (≥10–12 MPa) |
| Beam sides | 24 – 36 h |
| Slab soffit panels | 72 – 96 h |
| Back-propping | 2 layers for 7–14 days |

Assumptions & Exclusions

- Excludes: Ready-Mix Concrete supply/transport, reinforcement fabrication/placing, external scaffolding (beyond MIVAN edge protection), tower crane ownership, curing water, embedded MEP.

- Basis: 150–200 reuse cycles of aluminium panels; stable 4–6 day floor cycle; trained crews.

# Internal Room Plumbing & Sanitary Module – Full Material & Labour Consumption

Description:
This chart provides a comprehensive overview of all materials, fittings, fixtures, consumables, and labour required for concealed plumbing and sanitary installations inside the room, for different bathroom module types. It covers supply and waste piping, traps, valves, wall sleeves, stub-outs to service duct (inside room only), sanitary fixtures, adhesives, sealants, and labour, with allowances for making-good, pressure testing, and wastage. This table is designed for Total Sheet module-wise BOQ generation and can be scaled per number of modules in a project.

| Item / Labour | Single (Basin) | Double (Basin + WC) | Triple (Basin + WC + Shower) | All-in-One (Basin + WC + Shower + Bathtub) |
| --- | --- | --- | --- | --- |
| Wash Basin | 1 | 1 | 1 | 1 |
| Basin Tap | 1 | 1 | 1 | 1 |
| WC with Cistern / Flush Valve | 0 | 1 | 1 | 1 |
| Health Faucet + Hose | 0 | 1 | 1 | 1 |
| Shower Mixer / Diverter | 0 | 0 | 1 | 1 |
| Shower Arm + Head | 0 | 0 | 1 | 1 |
| Bathtub + Mixer / Overflow / Waste Set | 0 | 0 | 0 | 1 |
| Angle Valves ½″ (nos.) | 1 | 2 | 3 | 4 |
| Bottle Trap (nos.) | 1 | 1 | 1 | 1 |
| Floor Trap (nos.) | 1 | 1 | 2 | 3 |
| CPVC Pipe ½″ (Supply) | 6 m | 10 m | 15 m | 16 m |
| CPVC Pipe ¾″ (Supply – Hot/Cold) | 0 | 0 | 0 | 4 m |
| CPVC Elbows ½″ (nos.) | 4 | 6 | 8 | 10 |
| CPVC Tees ½″ (nos.) | 1 | 3 | 4 | 5 |
| CPVC Unions ½″ (nos.) | 0 | 1 | 1 | 1 |
| PVC Waste Pipe 50–150 mm | 3 m Ø50 | 5 m Ø75 | 7 m Ø100 | 10 m Ø100/150 |
| Wall Sleeves (nos.) | 1 | 2 | 3 | 4 |
| PVC Waste Elbows / Tees / Reducers (nos.) | 2 | 3 | 4 | 5 |
| P-Trap (nos.) | 0 | 1 | 1 | 1 |
| Pipe Stub-Out to Duct (Supply) (m) | 0.5 | 0.7 | 0.9 | 1.2 |
| Pipe Stub-Out to Duct (Waste) (m) | 0.5 Ø50 | 0.7 Ø75 | 0.9 Ø100 | 1.2 Ø100/150 |
| Flexible Coupling (nos.) | 1 | 1 | 1 | 1 |
| Reducer / Transition (PVC→Riser) (nos.) | 0 | 1 | 1 | 1 |
| Isolation / Stop Valve at Stub-Out (nos.) | 1 | 1 | 1 | 1 |
| Riser / Duct Escutcheon Plate (nos.) | 1 | 1 | 1 | 1 |
| Fire-stop / Intumescent Seal (tube/putty) | 1 small | 1 small | 1 small | 1 kit |
| Access Panel / Cleanout Plate (nos.) | 0 | 1 | 1 | 1 |
| Pipe Support / Clamp (nos.) | 1 | 2 | 2 | 3 |
| Rubber Gasket / Flange (nos.) | 0 | 1 | 1 | 1 |
| Mortar / Finishing Materials (kg) | 0.5 | 1.0 | 1.5 | 2.0 |
| Sealing & Waterproofing Tape / Mastic (m) | 1 | 1.5 | 2 | 2.5 |
| Solvent Cement for Pipes (ml) | 100 | 150 | 200 | 250 |
| PTFE / Teflon Tape (rolls) | 0.5 | 1 | 1.5 | 2 |
| Silicone Sealant Tube | 0.25 | 0.5 | 1 | 1 |
| Fasteners / Anchors / Screws (sets) | 1 | 2 | 3 | 4 |
| Plumber – Supply & Drain Installation (man-hrs) | 2 | 3 | 4.5 | 6 |
| Helper – Installation & Cleanup (man-hrs) | 2 | 3 | 4 | 5 |
| Wall Chasing & Making-Good (m²) | 1.5 | 2.5 | 4 | 5 |
| Pressure Testing / Commissioning (man-hrs) | 0.5 | 0.75 | 1 | 1.25 |
| Wastage / Contingency on Pipes & Fittings (%) | 5% | 5% | 7% | 7% |

This report is prepared by UTNA SYSTEMS PVT LTD, integrating standard industry practices, Indian construction codes, and proprietary data insights from the Total Sheet platform. The material and manpower consumption metrics are indicative and may vary based on project scale, region, and methodology. For real-time estimation and BOQ automation, refer to https://totalsheet.app/

© 2025 Utna Systems Pvt Ltd. | All rights reserved. | Powered by Total Sheet – Construction Intelligence Platform.